02200000000801

1

2

9[...............................................................]0

MACHINE LANGUAGE COURSE PART V by Mark van den Boer

Logical Operations

Instruction:   AND

Syntax:        AND <ea>,Dn

AND Dn,<ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V,C  always cleared

Addressing modes:

Source: (destination is Dn)

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Destination: (source is Dn)

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform   a   boolean  bitwise  AND   from   source   to

destination.

Examples:

Instruction              Before         After

AND.B d0,d1              d0=33333333    d0=33333333

d1=ffffffff    d1=ffffffcc

AND.W d0,d1              d0=33333333    d0=33333333

d1=ffffffff    d1=ffffcccc

AND.L d0,d1              d0=33333333    d0=33333333

d1=ffffffff    d1=cccccccc

Instruction:   ANDI

Syntax:        ANDI #,<ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V,C  always cleared

Addressing modes:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

CCR, SR

Function: Perform   a   boolean  bitwise  AND   from   source   to

destination.

Examples:

Instruction              Before         After

ANDI.B #7,d0             d0=33333333    d0=33333330

ANDI.W #$ff00,SR         SR=ffff        SR=ff00

Instruction:   EOR

Syntax:        EOR Dn,<ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V,C  always cleared

Addressing modes:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform  a bitwise exclusive or from source to  destina-

tion.  You  will notice that this instruction  can  only

take a data register as the source,  unlike the AND  and

OR  instructions.  I  think the designers of  the  68000

should have mode addressing modes more orthogonal.

Examples:

Instruction              Before         After

EOR.L d0,d1              d0=ffffffff    d0=ffffffff

d1=f0f0f0f0    d1=0f0f0f0f

Instruction:   EORI

Syntax:        EORI #,<ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V,C  always cleared

Addressing modes:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

CCR, SR

Function: Perform   a   bitwise  exclusive  or  from   source   to

destination.

Examples:

Instruction              Before         After

EORI.B #$ff,d0           d0=33333330    d0=333333cf

EORI.W #$00ff,SR         SR=ff32        SR=ffcd

Instruction:   NOT

Syntax:        NOT <ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V,C  always cleared

Addressing modes:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform a bitwise not to the effective address  operand.

Note that NOT.W d0 is the same as EORI.W #$ffff,d0.

Examples:

Instruction              Before         After

NOT.B #$ff,d0            d0=ffffff78    d0=ffffff87

Instruction:   OR

Syntax:        OR <ea>,Dn

OR Dn,<ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V,C  always cleared

Addressing modes:

Source: (destination is Dn)

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Destination: (source is Dn)

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform a boolean bitwise OR from source to destination.

Examples:

Instruction              Before         After

OR.B d0,d1               d0=ffffffff    d0=ffffffff

d1=33333333    d1=333333ff

OR.W d0,d1               d0=ffffffff    d0=ffffffff

d1=33333333    d1=3333ffff

OR.L d0,d1               d0=ffffffff    d0=ffffffff

d1=33333333    d1=ffffffff

Instruction:   ORI

Syntax:        ORI #,<ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V,C  always cleared

Addressing modes:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

CCR, SR

Function: Perform  a boolean bitwise OR from source  to  destina-

tion.

Examples:

Instruction              Before         After

ORI.B #7,d0               d0=33333330    d0=33333333

ORI.W #$00ff,SR           SR=ff00        SR=ffff

Originally published in ST NEWS Volume 2 Issue 4.