68040 Info:

----------------------------

This new CISC microprocessor

offers RISC performance

----------------------------

Motorola has officially unwrapped its newest 32-bit

microprocessor, the 68040. Manufactured with 0.8-micron

high-speed CMOS technology, the 68040 packs 1.2 million

transistors on a single silicon die. With 900,000 extra

transistors to work with over the 300,000 transistors in a 68000

processor, the 68040's designers added new features and boosted

performance. New features include the following:

-- Optimised 68030 integer unit. While retaining object-code

compatibility with previous 68000-family processors, the IU has

been optimised to execute instructions in fewer clock cycles

(i.e., run faster). The claimed boost in performance is three

times that of a 68030.

-- Integral FPU. The 68020 and 68030 require external FPU

coprocessor chips to handle floating-point math. The 68040,

however, has an FPU built into it, giving it the power to do

serious number crunching. The FPU's data types are compatible

with the ANSI/IEEE 754 standard for binary floating-point math,

and its instruction set is object code-compatible with Motorola's

68881/68882 FPUs. Like the IU, the 68040's on-chip FPU has been

optimised to execute frequently used instructions using fewer

clock cycles. The claimed performance boost is 10 times that of a

68882.

-- Large caches. Processor accesses to the system bus are

minimised by storing the most recently used set of instructions

or data in on-chip, 4K-byte caches. Both caches operate

independently but can be accessed at the same time. Bus snoop

logic is used to maintain cache coherence (i.e., it ensures that

the cache's contents match those parts of memory corresponding to

the cache). The bus snooper's design is fine-tuned to support

multiprocessor systems where one or more bus masters or 68040s

might share the same section of memory.

-- Separate memory units for instructions and data. Each memory

unit consists of a memory management unit, a cache controller,

and bus snoop logic. The MMUs use a subset of the 68030's MMU

instruction set. Both memory units function independently of each

other to improve processor throughput.

The 68040 ships with an initial clock speed of 25MHz; higher

speeds are to be available in the future, Motorola says. The

68040 comes in a 179-pin grid-array package. With the elimination

of coprocessor function lines (now that the MMU and FPU are

consolidated onto the processor) and the addition of snoop

control lines, the 68040 is not pin-compatible with the 68030.

Because of the 68040's software compatibility with its

predecessors, it can tap into the existing software base of 680x0

applications. It does this not only while eliminating a component

(the FPU) from a computer's design, but also while improving

performance. In fact, the 68040 executes instructions on the

average of nearly once per clock cycle -- the same as a RISC

processor.

Fine-Tuned for Performance

The 68040 was built on the firm foundation of its

predecessors. The design team used the experience garnered from

developing earlier processors to aid in optimising the throughput

of the 040.

The 040 was designed from the ground up, Motorola engineers

said. It incorporates a high degree of parallelism using a number

of internal buses. An internal Harvard architecture gives the

processor full access to both instructions and data. Both the IU

and FPU have separate pipelines and can operate concurrently. For

example, the FPU can perform floating-point instructions

independently of the IU. Each stream (instructions or data) has

its own dedicated cache and MU that function independently of

each other. A smart bus controller assigns priorities to bus

traffic to and from the caches.

There were several key areas where Motorola was able to

boost performance. The first was in reducing the clock cycles

needed to execute certain instructions. The next was to ensure

that the processor funnels instructions and data into itself

quickly and constantly, lest it stall while waiting on

information. The processor then gets its results back into the

system without interfering with incoming information. Finally, as

if this wasn't enough, the processor stays off the system bus to

a greater extent than is the case with other processor designs.

This lets DMA transfers and other bus masters have use of it.

CISC with the Speed of RISC

The IU was optimised so that high-usage instructions execute

in fewer clock cycles, particularly branch instructions. Motorola

said it performed thousands of code traces using real-world

applications to determine which instructions were used most

often. The IU consists of 6 stages: instruction prefetch, decode,

effective address calculation, operand fetch, execution, and

writeback (i.e., the result is written to either a register or to

memory). Each stage works concurrently on the instruction

pipeline. Dual prefetch and decode units deal with the branch

instructions: One set processes the instruction taken on the

branch, and another processes the instruction not taken. In this

way, no matter what the outcome, the IU has the next instruction

decoded and ready to go without seriously disrupting the

pipeline. This complex design has a big pay-off: Motorola has

determined that the average instruction takes 1.3 clock cycles to

execute. The ability to execute an instruction once per clock

cycle is the performance edge of RISC processors -- yet the

68040's IU accomplishes the same goal while executing

complex-instruction-set computer (CISC) instructions.

The FPU adds 11 registers to the 68040 register set: Eight

of them are 80-bit floating-point registers, and three are

status, control, and instruction address registers. The FPU has a

three-stage execution unit, and, like the IU, each stage operates

concurrently. Load and store instructions (FMOVE) can be

performed during other arithmetic operations, and a 64- by 8-bit

hardware multiplication unit speeds many calculations. However,

the FPU only implements a subset of the 68882 instructions

on-chip. The transcendental (trigonometric and exponential)

functions are emulated in software via a software trap. But

Motorola claims that even these instructions should execute 25%

to 100% faster on 25MHz 68040 than on a 33MHz 68882 FPU.

Boosting Throughput

In the area of throughput, each stream is managed by a

separate memory unit that uses an MMU for logical-to-physical

address translations during bus accesses. These MMUs support

demand-paged virtual memory. Both MMUs have a four-way

set-associative address translation cache (ATC) with 4 entries

(versus 22 entries for the 68030). The ATCs reduce processor

overhead by storing the most recent address translations. When an

address translation is required, the ATC is searched, and if it

contains the address, it is used immediately. Otherwise, a

combination of high-speed hardware logic and microcode searches

the translation tables located in main memory.

Like the PU, these MMUs implement a subset of the 68030's

MMU instruction set. Gone are the PLOAD and PMOVE instructions,

because enhanced existing instructions made them superfluous.

Also, only 2 memory page sizes are supported, 4K and 8K bytes,

whereas the 68030 MMU supported 8 page sizes ranging from 256

bytes to 32K bytes. A design tradeoff was made here: A

performance gain was possible by supporting only the 2 most

common page sizes. In any case, this change impacts only

operating-system code, since MMU instructions aren't normally

used by applications.

The two on-chip 4K caches improve processor throughput in 2

ways: They keep the pipelines filled and minimise system bus

accesses. To see how this is done, you must examine the structure

of the cache. Each is a four-way set-associative cache composed

of 64 sets of four lines. A line consists of 4 longwords, or 16

bytes. Cache lines are read or written rapidly using burst-mode

access (a type of bus transfer that moves 16 bytes in a minimum

of clock cycles). For read operations, this fills the cache

efficiently and, at the same time, loads adjacent instructions or

data into the cache that could be used in the near future.

Zen and the Art of Cache Maintenance

As the cache is accessed and data modified, cache-mode bits

in the ATC determine, on a page-by-page basis, the method by

which the information is handled. That is, the ATC entry that

corresponds to the address in main memory whose contents were

copied into the cache decides how the data will be updated. The

modes are cacheable write-through, cacheable copyback,

noncacheable, and noncacheable I/O.

In the cacheable write-through mode, an update to the data

cache forces a write to main memory. While this generates

additional bus activity, this mode is required when working with

a portion of memory that other processors share. The copyback

mode updates the cache line but without updating main memory. The

modified (or "dirty") cache line is copied back into main memory

only when absolutely necessary. "Noncacheable" indicates that the

data shouldn't be cached, which is typically the situation for

shared data structures or for locked accesses (e.g., an operand

access or a translation table entry update). Noncacheable I/O

indicates that the data can't be cached and must be read or

written in the exact order of instruction execution. This mode is

for memory-mapped I/O devices (typically a serial device) where

the information's order is crucial.

The bus snooper is used in multiple bus master situations

where a noncaching bus master, such as a DMA controller, might

modify the memory that is mapped into the 68040's cache. The bus

snooper monitors the external bus and updates the cache as

required.

Cache validity is handled on a line-by-line basis (i.e., a

cache miss triggers a burst-mode access that updates 16 bytes

either in the cache or main memory). The copyback mode minimises

writes to main memory, and the bus controller prioritises each

cache's external memory requests. Read requests take priority

over writes to ensure that the pipelines remain filled.

The caches are critical to the 040's overall throughput.

They keep instructions and data moving into the processor while

satisfying the apparently contradictory role of minimising system

bus accesses. Motorola estimates that the cache hit rate is about

93 percent for instruction and data reads and about 94 percent

for data writes.

A Processor for the 1990's

It is perhaps appropriate that Motorola has introduced the

68040 in the first month of the 1990s. The 040 has the power to

tackle the jobs with large amounts of information that we will be

dealing with regularly in the next ten or so years.

Preliminary results have a 68040 weighing in at 20 million

instructions per second versus the SPARC's 18 MIPS and the

80486's 15 MIPS, all clocked at 25MHz. On floating-point

operations, the 68040 antes up 3.5 million floating-point

operations per second versus the SPARCS's 2.6 MFLOPS and the

80486's 1 MFLOPS. If these numbers are accurate, then the 68040

already out performs one RISC processor.

But the computer industry doesn't stand still. As we move

into the new decade, we can expect new RISC processors to once

again take the lead in performance. Still, the 68040 shows that

owners of CISC systems can have their cake and eat it, too. They

don't have to forsake their software base or settle for mediocre

performance.

And Motorola is already working on the 68050.

|-THiS FiLE PASSED THR0UGH --- /\ ---.------ /\ ---*--.- FiDONET 2:200/612 --|

|                     .  * .  // \        . // \  .      FUJiNeT 7:102/102   |

| I.C.S Swedish HQ           //   \   +    //   \      .  MeGANeT 66:666/1   |

|                       +   //  /  \      //     \   +    NeST 90:1101/112   |

|  Sync World HQ           /\\  \\ /  .  //   \\ /                           |

|                      .  // \   \/     //    /\/   .    16800 DUAL STANDARD |

|  +46-451-91002          \\ /   /      \\  \/    +                          |

|                      *   \\   /  + .   \\  \ .    .  .                     |

|                        .  \\ /          \\ /                               |

|- SysOp: Troed ------------ \/ARCASTIC -- \/XISTENCE --- CoSysOp: Zaphod B -|

< Advertisment added using -=Bad Ad=- 1.91 by Troed/Sync. BBS: +46-451-91002 >