HOW TO RECEIVE APT PICTURES FROM THE NOAA SATELLITES

----------------------------------------------------

Courtesy Dallas Remote Imaging Group

Datalink RBBS =============> 214-394-7438

Dedicated to satellite tracking, decoding of NOAA/Soviet

meteorological satellite telemetry, and Digital Image

Processing of satellite pictures.

Jeff Wallach,      N5ITU, Chairman

John Williams,            Co-Chairman

Tom  Kelso,               Air Force/NASA liason

Jim Blocker,       KF5IW, Director Software Development

Jose Sancho       WB5YFU, Director of Engineering

All aspects of Amateur Radio covered on Datalink RBBS

(IF YOU COPY THIS BULLETIN FOR OTHER SYSTEMS, PLEASE BE KIND ENOUGH

TO LEAVE  THIS BANNER  AS DUE CREDIT TO THE HARD-WORKING FOLKS WHO

DEVELOPED AND CONTINUALLY UPDATE THIS INFORMATIVE 'HOW-TO'BULLETIN)

December 15, 1987

(rev 6)

Tom Gentry, K5VOU, has been kind enough to help prepare the following:

HOW TO RECEIVE APT PICTURES FROM THE NOAA SATELLITES

----------------------------------------------------

So  you  have  decided you want to receive the NOAA and Russian METEOR

orbiters  and  you  have  a  radio  that  receives  137.500  MHz and a

computer.  So  now what do you do? I hope this short dissertation will

steer you in the correct direction.

To  display a picture from the orbiting birds you need several things.

First  you  need  to  'acquire' the satellite. This means you need the

program to calculate its position at any time and determine if you can

hear  the  signals from the 'bird'. The best place to get this program

is  probably AMSAT-NA an organization dedicated to AMateur SATellites.

They have several very good tracking programs for the IBM and also for

Commodore  and  Apple as well. After you have acquired the program and

have it running on your computer, you then need the Keplerian Elements

for  the  NOAA  satellites.  These  elements  as well as some tracking

programs  are  available from the CELESTIAL RCP/M, run by TS Kelso, at

512-892-4180  (Austin, TX) and from the Weather Fax and Remote Imaging

DATALINK BBS run by Dr. Jeff Wallach, N5ITU as bulletins for some 50 +

sets of satellite  data.   Another bulletin also tells you to what the

printed  numbers  correspond  so  you can enter them into the computer

program.  Use  the  newest  elements  you  can  find  as  the orbiting

satellites are subject to forces which change their orbits. If you use

Keplerian Elements more than 3 or 4 weeks old, your chances of hearing

the bird are reduced.

Once you are tracking the birds you should hear them on your receiver.

They  will  be  on 137.62 (NOAA-9), 137.500 (NOAA-10), 137.850 (Meteor

2-14,2-15)  and  137.4  (Meteor  2-16).  Now  comes  the  first  thing

different  about  NOAA  orbiters.  The  frequency  deviation of the FM

transmission  is  about  +/-18-20  kHz.  This  is  4  times the normal

deviation  of  a police call and 1/3 the deviation of the FM broadcast

band   FM   signal.   If   you  receive  the  signal  on  the  regular

communications  width  of  the  scanner the light areas of the picture

will be noisy and the signal will sound distorted and the picture will

basically  be useless. If you use the FM broadcast bandwidth (assuming

your  scanner  will let you), the signal will be weak and there may be

too much background noise to see the picture. So what now ? Well there

are  two ways to fix the problem, find a receiver with the proper I.F.

bandwidth  filter  of  50  kHz,  or modify the I.F. of your scanner to

about  50-80 kHz. The simplest mod which has been found to be workable

is to remove the narrow communications I.F. filter and replace it with

a  0.01 uFd capacitor. This provides for good Wefax pictures from GOES

and  pretty  fair  NOAA  orbiter  pictures. It can of course result in

retuning  the  radio,  voiding the warranty and making the squelch not

work properly.

The  other  characteristic of the NOAA satellite transmissions is THEY

ARE  WEAK.  To  get  good  pictures  over a large area we have found a

pre-amplifier  to  be  essential. A GaAs-FET type can be obtained from

various  sources at a reasonable price but still about 50-100 dollars.

You  can  also build one for about 25 dollars from plans for a pre-amp

for the 2-meter amateur band and tune it to 137.5 easily.

Lastly   but   importantly,  the  antenna  needs  to  be  of  an  omni

directional,  uniform pattern type such as a turnstile antenna similar

to  those  used  in  the FM broadcast reception business but of course

tuned  to  137.5  MHz.  Mount  the pre-amp at the antenna if possible.

Crooked  coathangers  on  broomsticks have been known to work with the

pre-amps  mounted  on  the broomstick, so it is not difficult to build

the antenna.

Now that you have a nice audio signal from the satellite, what happens

next? The audio tone of 2400 HZ which is the carrier tone that is used

to  carry  the  picture  information (video), must be detected and the

video  data converted from analog to digital and then displayed on the

computer monitor by the software. The hardware and software to do this

is  available  from  several  sources  with more coming along. Several

stand-alone  boxes  are  also  available  that  produce  some  form of

computer  output either in printed form or on the display. In addition

the  people  on the DATALINK BBS may also have some data to share with

you regarding this equipment.

One  of  the  best  sources  for  information is the Weather Satellite

Handbook available from Dr. Ralph Taggart of Michigan State University

at 602 S. Jefferson, Mason, MI 48854 for $12.50. Taggart also writes a

monthly WEATHERSAT column which appears in '73 Magazine. A must is THE

JOURNAL OF THE ENVIRONMENTAL SALELLITE AMATEUR USERS' GROUP - JESAUG -

which is published quarterly. It is available for $12.00 per year from

the  editor,  Greg  Mengell, 2685 Ellenbrook Drive, Rancho Cordova, CA

95670. Back issues of the 'Journal' are also available.

At  the  present  the  simplest  software  for  the  IBM  is the Elmer

Schweittek MULTIFAX 2.0 software. A hardware kit is available from A&A

Engineering  which  includes  the  video  dectector  circuit  and  A/D

converter.  Input to the PC is via the game port. If you want to build

your  own,  complete documentation to build an interface circuit on an

IBM  PC  prototyping  board is available from H. Paul Shuch, N6TX, for

$5.00 - FAXBOARD.

The  WEPIX(tm)  software  and hardware should be available from Encomm

before  Christmas  time this year and it will produce pictures of 1200

lines  800  samples  of  each line displayed in 200 x 200 pixels in 16

shades  of  gray or 16 colors using an IBM PC with 640kB, EGA card and

at least one 360k disk.

'Typical' APT Receiving Station

-------------------------------

[1]   ANTENNA

-

[2]   PRE-AMP

-

-

-

-                          [ STAND ALONE UNITS ]

-

[3]   RECEIVER  ----------------->  FAX MACHINE  ---> PAPER

-                          WRASSE UNIT  ---> MONITOR

-                          YU3UMV UNIT  ---> MONITOR

-

-

-

[4] - VIDEO DETECTOR   -

-                  -

-                  -

[5] - ANALOG/DIGITAL   -

-                  -

-                  -            [ SCAN CONVERTERS ]

-                  -

[6] - INTERFACE TO PC  - <=======>  ROBOT 1200

-                  -            TAGGARTS'S WSH

-                  -            IMAGEWISE

-                  -

[7] -    SOFTWARE      -            ** SECOND DISPLAY **

-                  -               for weather images

-                  -

-                  -

[8] - GRAPHICS ADAPTOR -

-

-

-

-

[9]   PC DISPLAY / MONITOR

Display weather images

(or commands for SCAN CONVERTERS)

[10]   PC PRINTER / DOT MATRIX / LASER

NOTES:

------

[4] VIDEO DETECTOR, [5] ANALOG/DIGITAL CONVERTER, and

[6] INTERFACE TO PC may be in seperate hardware units

or all on one circuit board.

Sources:

----------------------------------------------------------------------

NOAA Data / Keplerian Elements:

DATALINK RBBS, N5ITU, Dr. Jeff Wallach, 214-394-7438

DALLAS REMOTE IMAGING GROUP ELECTRONIC BULLETIN BOARD

UPDATES ON NOAA BULLETINS, SATELLITE LAUNCHES, FREQUENCIES

DISPLAY PICTURES FROM NOAA SATS ON YOUR OWN P.C.

CELESTIAL RCP/M, TS KELSO, 512-892-4180

Tracking Software:

AMSAT-NA,  PO  Box  27, Washington, DC 20044, Software Exchange,

phone 301-589-6062 (voice)

DATALINK BBS, download SATTRAK0.ARC or ORBIT1-C.ARC

CELESTIAL RCP/M, TS KELSO, 512-892-4180, SATTRAK0.ARC and others

Receivers:

Vanguard Labs, 196-23 Jamaica, Hollis, NY 11423,

phone 718-468-2720

Hamtroincs, Inc.,65-D Moul Road, Hilton, NY 14468-9535

phone 716-392-9430

Crystals:

JAN  Crystals,  2400  Crystal  Dr,  Fort  Myers,  FL 33906-6017,

phone 1-800-237-3063 or 813-936-2397

Pre-Amps:

THL/Encomm, Inc, 1506 Capital Ave, Plano, TX 75074, phone 214-423-0024

Spectrum   International,Inc.,  Box  1084,  Concord  MA  01742,

phone 617-263-2145

Vanguard Labs, Hamtronics and GLB for Helical RF Filters for intermod

Antennas:

ARRL Handbook, 1986, turnstile over ground antenna.

ARRL Satellite Experimenter's Handbook by Martin Davidoff, K2UBC

Hardware Interface to PC:

A&A  Engineering,  2521  W. La Palma, Unit K, Anaheim, CA 92801,

phone 714-952-2114 - both AM dectector and A/D converter, input to PC

via game port (4-bits) or parallel I/O Interface (8-bits) - kit is

$50. - assembled and tested unit is $70.

Microcomm,  H. Paul Shuch, N6TX, 14908 Sandy Lane, San Jose, CA 95124,

phone 408-377-6137 - FAXBOARD - A Weather Facsimile Display Board for

the  IBM  PC  -  Complete  documentation  to build your own interface

circuit on an IBM PC prototyping board - $5.00

Video Detectors:

RTM  Circuit  Boards,  205  Elm  St.,Van  Horne,  IA 52346-0400,

video detector  only (the 'Wilson Board') see QST Magazine, Jan '86

QST Magazine, August 1985, Grant Zehr article, page 27, video detector

circuit

A/D Converter & Parallel I/O Interfaces for the IBM PC:

MetraByte,  440  Myles Standish Blvd., Taunton, MA 02780, Model PIO-12

board for the IBM PC, phone 617-880-3000

John  Bell  Engineering, Inc, 400 Oxford Way, Beimont, CA 94002,

phone 415-592-8411

Display Software:

Encomm,  Inc.  1506  Capital, Plano, TX 75074, WEPIX(tm) system,

phone 214-423-0024

Elmer Schwittek, New Address: 2347 Coach House Lane,  Naples, FL 33942,

Multifax 2.0, phone 813-434-2268

Clay Abrams Software, 1758 Comstock Lane, San Jose, CA 95124, software

for the ROBOT 1200 - see QST Magazine, Jan '86

Ralph  Taggart,  602  S. Jefferson, Mason, MI 48854, Weather Satellite

Handbook Scan Converter - for the Radio Shack CoCo3 - see 73 Magazine,

Nov '84 and Dec '84 and Handbook

Marta Systems, 209 W. Santa Paula St, Santa Paula, CA 93060,

phone 805-933-1270, 'Live From Space' for the AMIGA PC - $500.

ImageWise Display/Receiver, Circuit Cellar Inc. - Micromint Inc.,

4 Park Street, Vernon, CT 06066, phone 203-871-6170  -  BYTE Magazine,

May, Jun, Jul, Aug 1987. DATALINK can provide information on software

and video interface

Complete Systems - Hardware and Software:

Loren Johnson, P.O. Box 219, Cleveland, MN 56017, system for the

IBM-PC  -  $750, phone 507-931-4849

Greg Mengell, 2685 Ellenbrook Drive, Rancho Cordova, CA 95670,

APT Associates, phone 916-364-1572, Wraase System and FAX recorders

(Ask for DEPT. J. == Specify referral by Dallas Remote Imaging Group)

YU3UMV  Digital  Storage  and  Scan  Converter  for  Weather Satellite

Images,  described  in  VHF  Communications Magazine, Winter 4/82 and

Spring  1/83  - printed circuit baords and modifications described in

the Satellite Users' Group Journal - JESAUG - are also available from

Greg Mengell

Bibliography:

NOAA Satellite Predictions, see DATALINK Bulletins

NOAA APT NOTES, see DATALINK Bulletin

NOAA Bibliography, see DATALINK Bulletin

Summary of Soviet Meteorological Satellites, see DATALINK Bulletin

Wrasse Dedicated System, see DATALINK Bulletin

MULTIFAX 2.0, see review on DATALINK BBS - download MULTIFAX.DAT

Review DATALINK RBBS bulletins for more information.

DALLAS REMOTE IMAGING GROUP - DRIG ==============> 214-394-7438

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X