----------------------------------------------------------------

DSP32 and DSP32C Application Software Library  --  Version 2.2.1

----------------------------------------------------------------

*** File: README.DOC on distribution media ***

*** File: UPDATE.DOC appended, ~line 735   ***

The assembly-language routines from the DSP32 and DSP32C

Application Software Library are contained on these three

floppy disks, as well as one example program that

demonstrates the use of each routine.  The actual library

routines have the .ASM file extension, while the example

programs have the .S file extension.  All file names begin

with an underscore.  For example, the cosine routine is

contained in the file _COS.ASM, and the example program

demonstrating the use of this routine is in the file _COS.S.

To use a routine from this collection, simply append the

desired routine to your source file and then call the

routine with the required arguments. Each example program is

written in this form. I.e., a short "main" program with

needed routines appended.

The remainder of this file consists of short descriptions

of each routine (broken down by functional category) and

the Programmer's Guide chapter of the reference manual.

For a description of the changes made to the routines and

the documentation between Version 2.0 and 2.1, refer to the

file UPDATE.DOC.

1. DESCRIPTION OF THE ROUTINES

------------------------------

Arithmetic Routines (in alphabetical order)

-------------------------------------------

_acos     Calculates the arccosine (inverse cosine) function

of the argument, which must be in the range of

-0.966 to 0.966.

_alog10   Calculates the base 10 anti-logarithm of the

argument, which must be in the range of -38 to 38.

_alog2    Calculates the base 2 anti-logarithm of the

argument, which must be in the range of -127 to

127.999.

_aloge    Calculates the base e anti-logarithm of the

argument, which must be in the range of -88 to 88.

_arccos   Calculates the arccosine (inverse cosine) function

of the argument, which must be in the range of

-1.0 to 1.0.

_arcsine  Calculates the arcsine (inverse sine) function of

the argument, which must be in the range of -1.0

to 1.0.

_asin     Calculates the arcsine (inverse sine) function of

the argument, which must be in the range of -0.966

to 0.966.

_atan     Calculates the arctangent (inverse tangent)

function of the argument, which must be greater

than zero.

_cos      Calculates the cosine of the argument, which must

be an angle measured in radians and in the range

of -pi/2 to pi/2.

_div      Calculates the value of argument N divided by

argument D (divide routine).

_divf     Faster but less accurate divide routine.

_dsp32    Converts an array of numbers in the IEEE 32-bit

floating-point format to the internal DSP32/DSP32C

floating-point format.

_dsp32c   Converts an array of numbers in the IEEE 32-bit

floating-point format to the internal DSP32/DSP32C

floating-point format.  The maximum size of the

array is 2048.

_gran     Returns a random number with float value between 0

and 1.  The random numbers have a Gaussian

distribution with zero mean and unity variance.

_ieee32   Converts an array of numbers in the DSP32/DSP32C

floating-point format to the IEEE floating point

format.

_ieee32c  Converts an array of numbers in the DSP32/DSP32C

floating-point format to the IEEE floating point

format.  The maximum size of the array is 2048.

_inv      Calculates the inverse (reciprocal) value of the

argument.

_invf     Faster but less accurate inverse routine.

_invsqr   Calculates the inverse of the square root of the

argument.

_log10    Calculates the base 10 logarithm of the argument,

which must be greater than zero.

_log2     Calculates the base 2 logarithm of the argument,

which must be greater than zero.

_loge     Calculates the base e logarithm of the argument,

which must be greater than zero.

_ran      Returns a random number with float value uniformly

distributed between 0 and 1.

_ran24    Returns a random number with float value uniformly

distributed between 0 and 1.  The period of the

random number sequence is 16,777,216.

_sin      Calculates the sine of the argument, which must be

an angle measured in radians and in the range of

-pi/2 to pi/2.

_sqr      Fastest and most accurate square root routine.

_sqrt     Calculates the square root of the argument.

_sqrtf    Faster but less accurate square root routine.

_sqrtq    Fastest but least accurate square root routine.

_tan      Calculates the tangent of the argument, which must

be an angle measured in radians and in the range

of -pi/4 to pi/4.

_xtoy     Calculates the value of argument x to the power of

the argument y. The value of the argument x must

be greater than zero.

Matrix Routines (in alphabetical order)

---------------------------------------

_mat2x2   Multiply two 2x2 matrices.

_mat3x3   Multiply two 3x3 matrices.

_mat4x1   Multiply a 4x4 matrix by a 4x1 matrix.

_mat4x1f  Faster multiplying routine for a 4x4 matrix by a

4x1 matrix.

_mat4x4   Multiply two 4x4 matrices.

_mat5x5   Multiply two 5x5 matrices.

_matin2   Calculates the inverse of a 2x2 matrix.

_matinf   Faster but less accurate (than _matinv) matrix

inverse routine.

_matinv   General-purpose square matrix inversion routine.

_matmul   Multiply two general matrices, A and B, and stores

result in matrix C. The dimension of matrix A is

m x n, B is n x p, and C is m x p.

Filter Routines (in alphabetical order)

---------------------------------------

_dnsamp   Implements the sampling rate conversion when the

ratio of the new sampling frequency to the original

sampling frequency is between 0.5 and 1.0 (DSP32C only).

_dnsamp3  Implements the sampling rate conversion when the

ratio of the new sampling frequency to the original

sampling frequency is 1/3 (DSP32C only).

_fir      Calculates the output of the finite impulse

response (FIR) filter.

_fir5     Calculates the output of the finite impulse

response (FIR) filter where filter length must be

an integer multiple of 5.

_firc     Calculates the output of the complex finite

impulse response (FIR) filter.

_iir      Calculates the output of an infinite impulse

response (IIR) filter. The IIR filter corresponds

to the direct form II cascade of second-order

sections with four multiplications per section.

_iir2     Calculates the output of a 2-section, 4-multiply

per section infinite impulse response filter.

_iir3     Calculates the output of a 3-section, 4-multiply

per section infinite impulse response filter.

_iir4     Calculates the output of a 4-section, 4-multiply

per section infinite impulse response filter.

_iird     Calculates the output of an infinite impulse

response (IIR) filter. The IIR filter corresponds

to the direct form II cascade of second-order

sections with five multiplications per section.

_iirdN    Calculates the output of an infinite impulse

response (IIR) filter. N indicates the number of

sections, which can be 1, 2, 3, or 4.  The IIR

filter corresponds to the direct form II cascade

of second order sections with five multiplications

per section.

_iirt     Calculates the output of an infinite impulse

response (IIR) filter. The IIR filter corresponds

to the direct form I cascade of second order

sections with five multiplications per section.

_iirt1    Calculates the output of a 1-section, 5-multiply

per section (direct form I) IIR filter.

_iirt2    Calculates the output of a 2-section, 5-multiply

per section (direct form I) IIR filter.

_iirt3    Calculates the output of a 3-section, 5-multiply

per section (direct form I) IIR filter.

_iirt4    Calculates the output of a 4-section, 5-multiply

per section (direct form I) IIR filter.

_osc      Returns a sample value from a digital sinusoidal

oscillator.  The oscillator must first be

initialized by the routine _oscinit.

_oscN     Returns N sample values from a digital sinusoidal

oscillator.  The oscillator must first be

initialized by the routine _oscinit.

_oscamp   Returns an estimate of the square amplitude of a

digital oscillator from its state variables.

_oscinit  Initializes a digital sinusoidal oscillator.

After this initialization, the routine _osc can be

used to generate digital oscillator samples.

_tone     Returns a sample value from a digital sinusoidal

tone-pair.  The tone-pair must first be

initialized by the routine _toneini.

_toneN    Returns N combined sample values from a pair of

digital sinusoidal oscillators.  The oscillators

must first be initialized by the routine _toneini.

_toneamp  Returns an estimate of the square amplitude of a

pair of digital oscillators from the state

variables of the tone-pair.

_toneini  Initializes the start-up of a pair of digital

sinusoidal oscillators (tone).  After this

initialization, the routine _tone can be used to

generate tone-pair samples.

_upsamp   Implements the sampling rate conversion when the ratio

of the new sampling frequency to the original sampling

frequency is greater than 1 (DSP32C only).

_upsamp3  Implements the sampling rate conversion when the ratio

of the new sampling frequency to the original sampling

frequency is 3 (DSP32C only).

Adaptive Filter Routines (in alphabetical order)

------------------------------------------------

_lms      Implements the real, least-mean-square (LMS)

algorithm.

_lmsc     Implements the complex, least-mean-square (LMS)

algorithm.

_lmsl     Implements the leaky, real, least-mean-square

(LMS) algorithm.

FFT Routines (in alphabetical order)

------------------------------------

_chamm0   Multiplies an array of complex data by the Hamming

window.

_chann0   Multiplies an array of complex data by the Hanning

window.

_fft      Calculates the fast Fourier transform (FFT) of a

complex array. The size of the array must be a

power of 2, and the maximum array size is 4096.

_ffta     Calculates the fast Fourier transform (FFT) of a

complex array. The size of the array must be a

power of 2, and the maximum array size is 4096.

_ffta16k  Calculates the fast Fourier transform (FFT) of a

complex array. The size of the array must be a

power of 2, and the maximum array size is 16384.

Only for the DSP32C device.

_ffta1k   Calculates a complex 1024-point fast Fourier

transform (FFT).

_ffta512  Calculates a complex 64-, 128-, 256, or 512-point

fast Fourier transform (FFT).

_fftb     Calculates the fast Fourier transform (FFT) of a

complex array. The size of the array must be a

power of 2, and the maximum array size is 4096.

Separate arrays are used to store the real and

imaginary parts of the data.

_fftc     Calculates the fast Fourier transform (FFT) of a

complex array. The size of the array must be a

power of 2, and the maximum array size is 4096.

Separate arrays are used to store the real and

imaginary parts of the data.

_fftc16k  Calculates the fast Fourier transform (FFT) of a

complex array. The size of the array must be a

power of 2, and the maximum array size is 16384.

Separate arrays are used to store the real and

imaginary parts of the data.  Only for the DSP32C

device.

_fftc1k   Calculates a complex 1024-point fast Fourier

transform (FFT).  Separate arrays are used to

store the real and imaginary parts of the data.

_fftc512  Calculates a complex 64-, 128-, 256, or 512-point

fast Fourier transform (FFT).  Separate arrays are

used to store the real and imaginary parts of the

data.

_hamm     Multiply an array of real data by the Hamming

window.

_hamm0    Multiply an array of real data by the Hamming

window.  The length of the array must be a power

of 2, and the maximum array size is 4096.

_hamm1    Multiply an array of real data by the Hamming

window.  The length of the array must be a power

of 2 plus 1, and the maximum array size is 4097.

_hann     Multiply an array of real data by the Hanning

window.

_hann0    Multiply an array of real data by the Hanning

window.  The length of the array must be a power

of 2, and the maximum array size is 4096.

_hann1    Multiply an array of real data by the Hanning

window.  The length of the array must be a power

of 2 plus 1, and the maximum array size is 4097.

_ifft     Calculates the inverse fast Fourier transform

(IFFT) of a complex array. The size of the array

must be a power of 2, and the maximum array size

is 4096.

_ifftb    Calculates the inverse fast Fourier transform

(IFFT) of a complex array. The size of the array

must be a power of 2, and the maximum array size

is 4096.  Separate arrays are used to store the

real and imaginary parts of the data.

_rffta    Calculates the fast Fourier transform (IFFT) of an

array of real data. The size of the array must be

a power of 2, and the maximum array size is 8192.

Graphics/Imaging Routines  (in alphabetical order)

--------------------------------------------------

_grey     Converts an array of color pixels represented by

16 bits per pixel (5 bits per color) to an array

of pixels with a 5-bit grey scale.

_histeq   Implements the histogram equalization algorithm on

a grey scale image with each pixel represented by

a 5-bit value.

2.  PROGRAMMER'S GUIDE

----------------------

The WE DSP32 and DSP32C Application Software Library

contains a set of commonly used signal processing routines

written in DSP32/DSP32C assembly language.  Its purpose is

to provide DSP32/DSP32C users with efficient, fully tested,

and easy-to-use routines that can be readily incorporated

into application programs.

The reference manual provides the programmer with complete

information concerning the use of the application library

routines.  Each routine is provided with documentation to

explain what the routine does, how it is used, and an

example which can be used to test the routine.

The routines are divided into several general categories for

easy reference.  These categories include arithmetic

operations, matrix operations, filters and tone generators,

adaptive filters, FFT routines, and graphics/imaging

routines.  Particular algorithms or functions may include

generic as well as specific routines.  As an example, the

routine _matmul can be used to multiply a general m x n

matrix by an n x p matrix, whereas the routine _mat3x3 is a

specialized routine that can only be used to multiply two

3 x 3 matrices.  Generic routines are usually optimized in

program code size, and specific routines are optimized for

execution speed.  The choice between generic and specific

routines depends on the particular application and must be

decided on a case-by-case basis.

The remainder of this chapter is organized as follows:

* 2.1 Routine Documentation Description describes the format

used in documenting each routine.

* 2.2 Routine Calls describes the routine calling procedure.

* 2.3 Routine Calling Examples provides several illustrative

examples of routine calls.

2.1  ROUTINE DOCUMENTATION DESCRIPTION

The following items describe the standard format of the

routine documentation.

* ROUTINE NAME

The name of a routine always starts with an underscore,

followed by the routine mnemonic name.  For example,

_matinv is the name of the routine that finds the

inverse of a square matrix.  Following the routine name

is a short description of what the routine does,

including any advantages and/or disadvantages over

similar routines.

* SEE ALSO

A short list of similar or associated routines is shown

here.

* ALGORITHM

Describes the algorithm that has been implemented and

includes references to more information.

* USAGE

This item specifies how to call the subprogram, with

arguments listed.  It is preceded by comments that list

memory requirements for the routine, input and output

data storage, and local and static variables, as well

as the registers used within the routine.

Local variables are RAM locations used by the routine

for intermediate calculations.  When an application

program calls several library routines, a common RAM

area can be assigned to the local variables for all of

the routines.  The size of this RAM area equals the

maximum local storage required by any subroutine called

in the main program.  Static variables are RAM

locations required by the routine and must remain in

existence after the routine is exited.  An example of

static variables are the state variables (i.e.,

previous input samples) in a finite impulse response

(FIR) filter.  Separate RAM locations must be reserved

for the static variables of each called routine and for

calls to the same routine with differing data.

Within a routine, the local and static variables are

each stored in consecutive RAM locations.  The first

location of the local variables is always labeled

_localV.  The routine label for static RAM variables is

_nameV, where name corresponds to the mnemonic name of

the called routine.  Although these static variables

can be placed anywhere in the memory space,

recommendations are presented regarding where to place

them to minimize conflict wait states.

Note that each time a particular routine is invoked for

a different application, the user must assign different

labels to the static variables. For example, when using

the _iir routine to calculate three different filters,

three sets of static variables must be maintained.

The register contents used in the routine are not

preserved, so the main program must save and restore

any registers used by the called routine (if the

contents of the registers must be preserved across the

call).

* ARGUMENTS

The argument names, passing sequence, and data types

are described in this section.  Arguments are passed to

the called routine either by value or by pointer only.

The argument values are passed in the locations

following the nop after the call instruction.

* MEMORY BANK ALLOCATION

This item gives information on the memory bank location

for the routine code, input and output data storage,

and local and static variables to minimize the number

of conflict wait states generated when the routine is

executed.

Note: A "conflict wait state" is generated when

consecutive memory accesses (non-interleaved) are made

to the same physical memory device on the DSP32C (or

memory bank on the DSP32).  "External memory wait

states" are generated by the DSP32C when accessing slow

external memory devices.  Throughout this document, the

phrase "wait states" refers to conflict wait states.

Each wait state is always 1/4 of an instruction cycle.

For example, on a DSP32C with an instruction cycle time

of 80 ns a wait state is 20 ns long.

* PERFORMANCE

Information on the routine execution time and accuracy

of the result is given here.  Each DSP32 instruction

corresponds to four DSP32 clock cycles and each wait

state corresponds to one DSP32 clock cycle.  The

execution time is calculated as follows:

4 (#instructions) + #wait_states

Execution time (us) = --------------------------------

DSP clock in MHz

* EXAMPLE

A program example using the routine is given, along

with the results of its execution.

* ROUTINE SOURCE CODE

The complete source code for the routine is given here.

The comments included at the beginning of the source

code listing describe the form of the routine call, the

argument types, and the registers used by the routine.

2.2  ROUTINE CALLS

To use the DSP32/DSP32C routines, just append the selected

routine code to the application program (or assemble it

separately and link it to the main program).  The

appropriate memory locations for the input data, output

data, and local and static variables (if any) must also be

allocated.

The general format for calling a routine is:

call routine_name (r14)

nop

arg1, arg2, ...

The routines are called by their name, e.g., _cos, for the

cosine routine.  The return address register must be r14.

The call is followed by a "latent" instruction, either a

nop, or a meaningful instruction from the main program.  The

list of arguments then follows.  The arguments are passed to

the called routine either by value or by pointer only.  In

the latter case, the pointer value corresponds to the

address containing the argument value(s).  The argument list

may also contain pointers for the local and/or static

variables.

The argument section of each routine's documentation gives

the sequence and data type of each argument.

2.3  ROUTINE CALLING EXAMPLES

Three illustrative examples are given below.  In each

example, the main program calls one of the application

library routines, and then executes a write to the pir

register.  The program then waits to allow the result to be

examined.  To run these programs in the DSP32/DSP32C

simulator, a breakpoint should be set at the label, pause.

However, there is no need to set the breakpoint when these

programs are run on a DSP32 or DSP32C Development System.

It is easy to assemble and test a sample program.  Assuming

that the main program and any needed routines are contained

in a single source file, the program can be assembled and

linked using the command

d3make -N -m2 program.s

for the DSP32 device or the command

d3make -Q -m6 program.s

for the DSP32C device.  Of course, other specific options

may be required to run on various hardware configurations,

but these options are fine for the simulator or hardware

development systems.  Then, the program can be run on the

simulator using the command

d3sim -P program

The -P option is used to disable the simulation of

external memory wait states.

When using the DSP32C software development card, the two

example programs _ffta16k.s and _fftc16k.s must be linked

using the ifile mmode6.32c.  This ifile allows blocks of

data larger than 16 Kwords to be placed in memory section

B of the development card.  For example, to assemble and

link the example program _ffta16k.s, use the commands:

d3as -Q _ffta16k.s

d3ld mmode6.32c -o _ffta16k _ffta16k.s

2.3.1  Calling the _sin Routine

The following program calls the sine routine to evaluate

sin (pi/4) or sin (0.7853981635).  The input argument and

the returned value are stored in location in and out,

respectively.  The expected returned value is 0.7071068.

/* Example for calling the _sin routine ------------- */

#include <dspregs.h>

.global pause, in, out

main:  call _sin (r14)

nop

int24  in        /* argument 1 (pointer) */

int24  out       /* argument 2 (pointer) */

pir = r1

pause: if (pif) goto pause

nop

goto main

nop

in:    float 0.7853981635

out:   float 0.0  /* The expected result is 0.7071068 */

The include file dspregs.h allows programs to be written

using DSP32C keywords. Then, if the program is assembled for

the DSP32 device, these keywords are translated to the

corresponding DSP32 keywords. For example, if the example

above is assembled for the DSP32, the int24 directives are

translated to int directives.  If the file dspregs.h is

included, the following DSP32C keywords are translated when

assembling for the DSP32:

Translates to

The DSP32C keyword      the DSP32 keyword

------------------      -----------------

r1e -- r21e               r1 -- r21

int24                     int

float24                   float

2.3.2  Calling the _xtoy Routine

The _xtoy routine is called to evaluate the third root of

the number, 729,000,000.0.  The argument list consists of

a pointer to scratch pad memory locations (2 float values),

pointers to the values of x and y, and the location where

the result is to be stored.  The expected result is 900.0.

/* Example for calling the _xtoy routine ------------ */

#include <dspregs.h>

.global pause, x, y, out

main:  call _xtoy (r14)

nop

int24  _localV   /* arg 1 (pointer to local variables) */

int24  x, y, out /* arguments 2 to 4 (pointers) */

pir = r2

pause: goto pause

nop

_localV: 2*float 0.0

x:       float 729000000.0

y:       float 1/3.  /* One third */

out:     float 0.0   /* The expected result is 900.0 */

2.3.3  Calling the _fft Routine

The _fft routine is called to calculate the FFT of a complex

16-point array.  The arguments consist of the array size (16),

M (=4) where 2^M=16, and _fftIO, which is the starting

location for the array values.  The 32 array values (arranged

in 16 complex pairs) correspond to the values of a 1 kHz tone

sampled at 8 kHz.  Since an in-place FFT algorithm is used,

the output values are pointed to by _fftIO after the routine

is run.  The expected results at mem[_fftIO:_fftIO+4*15] are:

0.0000000  0.0000000 0.0000000 0.0000000

5.6564264 -5.6564279 0.0000000 0.0000000

0.0000000  0.0000000 0.0000000 0.0000000

-0.0004265 -0.0004265 0.0000000 0.0000000

The example program is shown below.

/* Example for calling the _fft routine ------------- */

#include <dspregs.h>

.global pause, _fftIO

main:    call _fft (r14)

nop

int24  16, 4         /* arguments 1 and 2 (values) */

int24  _fftIO        /* argument 3 (pointer) */

.align   4             /* for alignment */

pir = r3

pause:   goto pause

nop

_fftIO:  float  .707, 0.,  1., 0.,  .707, 0., 0., 0.

float -.707, 0., -1., 0., -.707, 0., 0., 0.

float  .707, 0.,  1., 0.,  .707, 0., 0., 0.

float -.707, 0., -1., 0., -.707, 0., 0., 0.

*** =================<  END OF FILE: README.DOC >===================== ***

-----------------------------------------------------------------

Routines added/modified for Version 2.1   (since 2.0)

-----------------------------------------------------------------

*** File: UPDATE.DOC on distribution media ***

Routine		Comments

-------		------------------------------------

_acos		Minor documentation update only

_arccos		Addition: New arc-cosine routine

_arcsin		Addition: New arc-sine routine

_asin		Minor documentation update only

_gran		Minor documentation correction only

_inv		Routine bug fix

_invsqr		Routine and example program bug fixes

_tan		Documentation correction only

_xtoy		Documentation correction only

_matin2		Documentation correction only

_tone		Documentation correction only

_chann0		Routine bug fix

_hann0		Routine bug fix

_ffta		Routine bug fix

_ffta16k	Routine bug fix

_ffta1k		Routine bug fix

_ffta512	Routine bug fix

_fftc		Routine bug fix

_fftc16k	Routine bug fix

_fftc1k		Routine bug fix

_fftc512	Routine bug fix

_rffta		Routine bug fix

*** A User Manual Addendum was printed for this Release ***

-----------------------------------------------------------------

Routines added/modified for Version 2.1.1

-----------------------------------------------------------------

Routine		Comments

-------		------------------------------------

_ran24		Routine code update

_toneN		Minor documentation update only

_toneini	Routine bug fix

_rffta		Routine bug fix

-----------------------------------------------------------------

Routines added/modified for Version 2.2

-----------------------------------------------------------------

Routine		Comments

-------		------------------------------------

_dnsamp		Addition: Sample rate conversion routine

_dnsamp3	Addition: Sample rate conversion routine

_upsamp		Addition: Sample rate conversion routine

_upsamp3	Addition: Sample rate conversion routine

*** The User Manual was updated for Release 2.2 ***

-----------------------------------------------------------------

Documentation affected

-----------------------------------------------------------------

In addition to the "man" pages for each of the new and updated

routines, the following items have been modified to reflect the

current status of the application library:

Overall Table of Contents

Table of Contents for Appendix A

Table of Contents for Appendix C

FFT Summary document

README.DOC file distributed with source files

UPDATE.DOC file (this file) distributed with source files

-----------------------------------------------------------------

Routines added/modified for Version 2.2.1

-----------------------------------------------------------------

Routine		Comments

-------		------------------------------------

_dsp32		Routine bug fix

_ieee32		Minor documentation update only