AT&T - Technologies                                    Document No. AP88-05DMOS

August, 1988

IMPLEMENTATION OF THE CCITT WIDEBAND CODER

WITH THE

WE(R) DSP16 DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

wide.doc    This document.

wide.s      Program Listing 1.  Wideband Coder Source Code, page 12.

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

CHANGES of wide.s with respect to paper version.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1. Delete these lines (~854,855, page 25):

stop:  goto loop       /* Go back to execute the next input    */

/* data                                 */

2. Add these lines to end (page 28):

MEL:   int

tem2:  int

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1988 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

CONTENTS

~~~~~~~~~

Introduction			(included below)

Algorithm			(included below)

Transmitter

Transmit Quadrature Mirror Filters

Encoder

Receiver

Decoder

Receive Quadrature Mirror Filters

Wideband Coder Implementation

Hardware Considerations

Software Considerations

References			(included below)

Source Code			(separate file)

INTRODUCTION

~~~~~~~~~~~~~

In 1986, the International Telegraph and Telephone Consultative Committee

(CCITT) Study Group XVIII recommended an international standard for coding

wideband speech and music (50 Hz -- 7000 Hz) at 64 kbits/s or less [1]*.

( * indicates a reference, listed at end. )

Some applications envisioned for such a coder are in teleconferencing and

commentary channels, and for use in high-grade telephones for integrated

services digital network (ISDN) applications.  The algorithm uses

quadrature mirror filters (QMF) to split the signal into two bands that

are encoded using adaptive differential pulse-code modulation (ADPCM).

The coder can operate at 64 kbits/s (with a 4 kHz bandwith rate),

56 kbits/s, and 48 kbits/s.

This application note describes the implementation of this algorithm on the

WE DSP16 Digital Signal Processor and an AT&T T7520 Codec. Details of the

CCITT recommendation are described, followed by a description of the

implementation and testing of the coder.

ALGORITHM

~~~~~~~~~~

The coding algorithm is a sub-band coder with two 4 kHz bands.  Each band

is encoded using an ADPCM coder.  The upper band is always quantized to 2

bits.  The lower band is assigned 6, 5, or 4 bits, depending on the desired

speech coding rate.  The CCITT recommendation includes a 60-level quantizer

that avoids certain bit combinations (such as the all zeros case). Since

each bit in the encoded bit stream contributes 8 kbits/s of the overall

rate, the least significant bit is discarded to obtain a 56 kbits/s rate.

The two least significant bits are discarded to obtain the 48 kbits/s rate.

The lower rates can be used to allow simultaneous transmission of a data

channel of 8 kbits/s or 16 kbits/s, respectively, on a 64 kbits/s channel

(such as an ISDN B channel).

The input signal to the transmitter is sampled at 16 kHz and filtered by

a 24th order QMF bank to split the signal into a lower 4 kHz and upper

4 kHz band that are sampled at 8 kHz.  Using an ADPCM encoder, the upper

band is then encoded at 2 bits/sample, and the lower band at 6 bits/sample.

The ADPCM algorithm is the same used in the CCITT 32 kbits/s coding

standard [2].  The one or two least significant bits of the encoded lower

band are then dropped to achieve either a 56 kbits/s or 48 kbits/s rate.

The encoded bits are multiplexed with the most significant bits and the

upper band transmitted first.  Inverse operations are performed in the

receiver to reconstruct the original signal.  The QMF filter coefficients

are the same for both directions, while different quantization tables are

used by the receiver for the three lower-band bit rates.

REFERENCES

~~~~~~~~~~~

1. "7 kHz Audio Coding Within 64 kbits/s," CCITT Draft Recommendation G.72x,

CCITT Study Group XVIII, Report of Working Party XVIII/8\f1, July, 1986.

2. "32 kbits/s Adaptive Differential Pulse Code Modulation (ADPCM),"

CCITT Study Group XVIII, Working Party 8, Draft Revision of

Recommendation G.721, Temporary Document No. D.723/XVIII,

Source: U.S.A., Geneva, July, 1986.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=