AT&T Technologies                                    Document No. AP88-08DMOS

June, 1988

DUAL-TONE MULTIFREQUENCY RECEIVER

USING THE

WE(R) DSP32 DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

dft-dtmf.doc 	This document.

dft-dtmf.s 	Program Listing 1.  DTMF Receiver Source Code, Page 7.

Auxilary Files

==============

mdtmf3.s	|

sdtmf31.s	|  These are the latest versions of the DTMF code.

sdtmf31.ex	|  See under USAGE NOTES for a description on how to

sis1.d		|  use these files.

sdtmf32.s	|

NOTICE

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1988 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

CHANGES of dft-dtmf.s with respect to paper version.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

(1) Line ~20:

begin:  if (syc)  goto begin    /* wait for sync to go high before      */

change to

begin:  if (sys)  goto begin    /* wait for sync to go low before	*/

(2) Line ~174:

lrgmag4:a1=*r3+

change to:

lrgmag4:a1=*r3++

(3) Other differences are in format only.

ASSEMBLY And LINK

~~~~~~~~~~~~~~~~~~

d3make -N -W dft-dtmf.s

===================

Auxilary Code Files

===================

The following files for version 3.1 of the DFT based DTMF receiver are

described in this section:

sdtmf31.s

sdtmf31.ex

sis1.d

sdtmf32.s

mdtmf3.s

The performance of the algorithm has been improved by adding a total

energy ratio test, reducing the frame length from 206 to 102 and by fine

tuning some of the ratio coefficients.  Now only 5 errors are detected

when talkoff is tested with the mitel tape and the other test results

basically remain the same.

These notes are designed to take you from simulating 1 DTMF receiver

to testing 16 DTMF receivers in real time.

sdtmf31

~~~~~~~~

sdtmf31.s implements 1 DTMF receiver.  When a valid digit is detected it

is written to the integer RAM location called output.  All succeeding

digits are appended to this location.  RAM is initialized to store a

maximum of 200 valid digits.  The assembled code (sdtmf31) can be run

and tested in either the simulator or the development system.

To test sdtmf31 in the simulator you need to create an external input

file of 8 bit mu-law samples of touchtone signals.  Since data is read

into the DSP32 through the serial input port you also have to set up

the serial input clocks (pild and pick).  To get you started I have

included files sis1.d and sdtmf31.ex to take you through a sample

simulation.  sis1.d contains 4 50ms touchtone digits (1,5,9,13) each

separated by 50ms of silence.  sdtmf31.ex contains all of the commands

needed to set up the serial input clocks, identify the input file,

set up a breakpoint and display the locations in RAM that contain the

digits (1,5,9,13) when they have been validated.

PLEASE NOTE:

IF YOU MODIFY THE CODE BY ADDING MORE LOGIC, THE PILD CLOCK SET UP

IN sdtmf31.ex MAY HAVE TO BE DECREASED.

Since the simulator runs much slower than the actual chip, if the pild

was set to 8Khz (since sdtmf31.s only implements 1 receiver) it would

take a very long time to run this simulation.  Therefore the pild clock

speed was increased so that each sample arrives right after the worst

case loop.  Before you execute this sample simulation note that the

input file contains 2900 samples, therefore, it will take from 1 to 15

minutes to complete the simulation and detect and then display the RAM

locations containing the valid digits (depending on the speed of your

computer).  To run this sample simulation you must have sdtmf31,

sdtmf31.ex, and sis1.d in your present directory and execute the

following command:

d3sim -e sdtmf31

After approximately 875,591 machine cycles, your output will appear as:

output[0:6]=

0x0000: 1 5 9 13

The file sdtmf31.ex sets a breakpoint at pause which will be hit after 28

frames (2900 samples) have been received.  sdtmf31 can also be executed

on the development system by typing the following commands:

d3sim -d1 -m3 sdtmf31

ioc=0x547

autowait=0

run

While the program is running you can interactively read RAM starting at

output to see if you have detected any digits in real time.

sdtmf32

~~~~~~~~

sdtmf32.s is the same as sdtmf31.s except that it writes valid digits to

the lower 8 bits of pdr instead of to RAM.  When a digit is written to

pdr the pdf flag goes high.  When pdr is read, pdf goes low.  To execute

sdtmf32 on the development system, type the following:

d3sim -d1 -m3 sdtmf32

ioc=0x547

autowait=0

run

mdtmf3

~~~~~~~

mdtmf3.s implements 16 DTMF receivers on 1 25MHz DSP32.  When a valid

digit is detected it is written to the lower byte of the pdr.  The

lower 4 bits contain the digit detected and the upper 4 bits contain

the receiver number.  At worst case you have 40 usec to read the pdr

before it is overwritten by another receiver that has detected a digit.

DMA is used to control the serial input.  Therefore, to test mdtmf3 you

must have up to 16 external codecs and some external hardware that

creates ick, ild, and sync.

mdtmf3.s was designed by optimizing sdtmf32.s for 1 channel and then

adding the extra control instructions to make it 16 channels.

Therefore, it may be useful to refer to sdtmf32.s to understand the

program logic.  If any changes are made to the existing program logic

it must be assembled as shown here:

d3make -m3 mdtmf3.s

All of this information is explained in detail in the enclosed application

note, "DTMF Receiver Using The Discrete Fourier Transform".

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

INDEX

~~~~~~

Introduction         (included below)

DTMF Requirements    (included below)

The Basic Algorithm  (included below)

Implementation

Performance

Hardware Description

Source Code

References           (included below)

INTRODUCTION

~~~~~~~~~~~~~

A common signaling method used in the telephone network is dual-tone

multifrequency (DTMF) signaling. In this scheme, pairs of tones are

used to signal the digits 0 through 9, pound (#), star (*), and the

digits A, B, C, and D. For each pair, one of the tones is selected

from a low group of four frequencies, and the other from a high group

of four frequencies. The correct detection of a digit requires both a

valid tone pair and the correct timing intervals.

DTMF signaling is used both to set up a call and to control features

such as call forwarding and conference calling. In some applications,

it is necessary to detect DTMF signaling in the presence of speech, so

it is important that the speech waveform is not interpreted as a valid

signaling tone.  Standard test procedures have been published [1] to

verify the performance of DTMF receivers for valid tones and for the

recognition of speech signals as tones.

The implementation of a DTMF receiver involves the detection of each

of the signaling tones, validation of a correct tone pair, and timing

to determine that a digit is present for the correct amount of time

and with the correct spacing between tones. In addition, depending on

the algorithm used to detect frequencies, it is sometimes necessary

to perform additional tests to improve the performance of the decoder

in the presence of speech. Current DSP technology allows several DTMF

receivers to be implemented on a single device. A DSP implementation

is useful in applications in which the digitized signal is available

and several channels need to be processed, such as in a private branch

exchange (PBX), or where other functions can be included on the same

device.

This application note describes a DTMF receiver based on the discrete

Fourier transform (DFT). Using this algorithm, 16 DTMF receivers can

be implemented on a WE DSP32 Digital Signal Processor using internal

RAM (for both data and program memory) and executing at a 160 ns

instruction cycle time.  The input data is assumed to be u-law coded,

and is received on the DSP32 serial interface. Detected digits can be

read from the DSP32 parallel port.

DTMF REQUIREMENTS

~~~~~~~~~~~~~~~~~~

Figure 1 shows the matrix of frequencies used to encode the 16 DTMF

symbols. Each symbol is represented by the sum of the two frequencies

that intersect the digit.  The row frequencies are in a low band,

below 1 kHz, and the column frequencies are in a high band, between

1 kHz and 2 kHz. The digits are displayed as they would appear on a

telephone's 4x4 matrix keypad (on standard telephone sets, the fourth

column is omitted). DTMF receivers are required to detect frequencies

with a tolerance of +/- 1.5 percent as valid tones.  Tones that are

offset by +/- 3.5 percent, or greater, must not be detected.  This

requirement not to detect tones is necessary to inhibit the detector

from falsely detecting speech and other signals as valid DTMF digits.

The receiver is required to work with a worst-case signal-to-noise

ratio (SNR) of 15 dB, and with an attenuation of 26~dB.

Another requirement is the ability to detect DTMF signals when the

two tones are received at different levels. The high-band tone may

be received at a lower level than the low-band tone due to the

attenuation characteristics of the telephone network. This level

difference is called twist, and the situation described is called

normal twist. Reverse twist occurs when the low-band tone is

received at a lower level than the high-band tone. The receiver must

operate with a maximum of 8 dB normal twist and 4 dB reverse twist.

In addition to the frequency, noise, and twist requirements, the DTMF

signal must meet timing requirements for duration and spacing of digit

tones.  Digits are required to be transmitted at a rate of less than

ten per second.  A minimum spacing of 50ms between tones is required,

and the tones must be present for a minimum of 40ms. Any tone-detection

scheme used to implement a DTMF receiver must have a significant time

resolution to verify correct digit timing.

A final requirement for the receiver is that it operates in the presence

of speech without incorrectly identifying the speech signal as a valid

DTMF symbol.  This is referred to as talk-off performance. Although this

requirement is not stated in strict numerical terms, standard recordings

such as the Mitel DTMF test tape [2] contain speech segments that are

used to test the receiver's performance under these conditions.

The remainder of this application note describes the algorithm,

implementation, and performance of the DFT-based WE DSP32 Digital

Signal Processor DTMF receiver.

THE BASIC ALGORITHM

~~~~~~~~~~~~~~~~~~~~

The general approach taken by this algorithm for DTMF tone detection

is to take the Fourier transform of the observed signal and search

for energy at the frequencies of interest.  Since the algorithm is

implemented on a DSP, a Discrete Fourier Transform (DFT) is used.

The analysis frame must be long enough to resolve the DTMF frequencies,

but short enough to detect the minimum length tone. A 12.75 ms frame

at a sampling rate of 8 kHz is a good choice [3].

To guard against talk-off, the total energy is calculated and compared

to the sum of the largest magnitudes in the high- and low-frequency

bands. If a valid tone pair is being transmitted, then the two values

should be equivalent.  But if there is speech present, the total

energy should be much greater.

The algorithm also exploits the rich harmonic structure of the speech

signal to assist in guarding against talk-off.  If speech has energy

at a frequency, f1, it most likely has significant energy at twice

the frequency, 2(f1).  Simulation of a DTMF tone by speech is prevented

by calculating the energy at each tone and at its second harmonic.

If the second harmonic is below a threshold proportional to the

fundamental, a pure tone is present.  Otherwise, it is assumed

that a speech signal is present.

In calculating the DFT, the Goertzel algorithm [4], a method for

calculating any single coefficient of a DFT, is chosen over a fast

Fourier transform (FFT) algorithm.  There are two reasons for this.

In order to obtain the required frequency resolution at an 8 kHz

sampling rate, a 256-point FFT would be required. Since the

algorithm for tone detection requires knowledge of the energy at

only 16 frequencies, it is more efficient to execute the Goertzel

algorithm for these 16 frequencies than to perform the FFT for all

256 frequencies.  In addition, the Goertzel algorithm is recursive,

eliminating the need to store 256 samples for the FFT for each DTMF

detector.  This saves both real-time and data memory.

>> Remainder of section shows equations with related text <<

>>   which are not easily shown in a plain ASCII file.    <<

REFERENCES

~~~~~~~~~~~

[1] Touch-Tone Calling - Requirements for Central Office,

AT&T Compatibility Bulletin No. 105, August 8, 1975.

[2] Tone Receiver Test Cassette #CM7291, Mitel Technical Data Manual,

Mitel Semiconductor, 2321 Morena Blvd. Suite M, San Diego, CA 92110.

[3] Mock, Patrick, "Add DTMF Generation and Decoding to DSP -uP Designs,"

EDN, March 21, 1985.

[4] Oppenheim, A.V. and Schaeffer, R.W., "Digital Signal Processing,"

Prentice-Hall, Englewood Cliffs, NJ, 1975.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=