AT&T Technologies                                    Document No. AP88-09DMOS

April, 1988

IMPLEMENTATION OF DIGITAL FILTERS

WITH THE

WE(R) DSP32 DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

fir32_1.txt   Program 1. FIR Routine 1 (fir),  p 8.  (example,not for assembly)

fir32_2.s     Program 2. FIR Filter Example,   p 13.

fir32_3.txt   Program 3. FIR Routine 2 (firF), p 14. (example,not for assembly)

fir32_4.txt   Program 4. FIR Routine 3 (fir5), p 15. (example,not for assembly)

iir32_5.txt   Program 5. IIR Routine 1 (iir),  p 18. (example,not for assembly)

iir32_6.s     Program 6. IIR Filter Example,   p 21.

iir32_7.txt   Program 7. IIR Routine 2 (iir2), p 21. (example,not for assembly)

iir32_8.txt   Program 8. IIR Routine 3 (iirt), p 22. (example,not for assembly)

iir32_9.txt   Program 9. IIR Routine 4 (iirt3),p 25. (example,not for assembly)

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1988 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

INDEX

~~~~~~

Introduction                   (included below)

Overview of Digital Filtering  (included below)

Digital FIR/IIR Filters

Practical Considerations in Digital Filter Implementation

WE DSP32 DSP Routine Coding Conventions

WE DSP32 DSP Routines for FIR Filters

WE DSP32 DSP Implementation of IIR Digital Filters

Prototyping Digital Filters With the WE DSP Development System

Conclusions       (included below)

References        (included below)

INTRODUCTION

~~~~~~~~~~~~~

Filtering is one of the most commonly used operations in signal processing.

In many applications, the use of digital filters is preferred to analog

filters. Among the advantages of digital filters are:

* Digital filters can be designed to meet more stringent requirements

* Digital filters offer high reliability, high performance, and high accuracy

* Digital filter characteristics do not change with temperature and time

* Programmable digital filters can be easily modified to meet changing filter

characteristics.

Implementation of digital filters, however, was difficult and expensive

until the recent advances in VLSI technologies and the availability of

programmable single-chip digital signal processors.

In this application note, digital filtering is reviewed first from the

concepts of the more familiar continuous time domain analog filters.

Described next are the two common types of digital filters, the finite

impulse response (FIR) and the infinite impulse response (IIR) filters.

This is followed by a discussion of the practical considerations on the

implementation of these filters and, in particular, the advantages of

using floating-point processors over fixed-point processors.  The

remainder of the application note covers the major topic of implementing

these digital filters with the WE DSP32 Digital Signal Processor [1].

The DSP32 is a single-chip 32-bit floating-point programmable digital

signal processor that is capable of executing 6 million instructions

per second, or 12 million floating-point operations per second with a

25 MHz DSP32 input clock.  These same filter routines can also be used

with the DSP32C Digital Signal Processor which is capable of executing

12.5 million instructions per second, or 25 million floating-point

operations per second with a 50 MHz input clock.  Complete FIR/IIR

filtering routines using the DSP32 are given. These routines are chosen

for their efficiency in minimizing memory requirement or execution time.

The routines are written in such a way so as to facilitate their

incorporation with a minimum of effort into the users' application

programs. Readers who have prior knowledge of digital filtering can

go directly to the sections on the DSP32 implementation.

OVERVIEW OF DIGITAL FILTERING

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

A system for digital filtering of analog signals is shown in Figure 1.

The incoming analog signal is first passed through an analog lowpass

filter, known as an anti-aliasing filter. This filter bandlimits the

frequency components of the input signal to less than one-half of the

sampling rate in order to minimize errors due to frequency aliasing.

The filtered signal is then sampled by the A/D converter to produce a

sequence of quantized samples or digital words. The sequence of digital

words can be operated upon numerically by a digital processor (under

software control) to produce another sequence of digital words.  The

altered sequence of digital samples is converted back to an analog signal

by the D/A converter. The final analog lowpass filter, or reconstruction

filter, is needed to compensate for the characteristics of the D/A

converter and to recover a smooth analog output signal.

The operation of the digital signal processor as described above

implements the system transfer function by converting an input sequence

into another sequence by means of some numerical computation algorithms.

The computation algorithms are called digital filters when the system

transfer function corresponds to filtering functions.

For more detailed discussion, readers are referred to the many

standard texts on digital signal processing [2,3].

CONCLUSIONS

~~~~~~~~~~~~

Three FIR filter routines and four IIR filter routines are given.

Tradeoffs between routine memory requirement and execution time are

discussed.  The characteristics of these routines are summarized in

Tables 1 and 2.  These routines can be readily incorporated into

DSP32 users' application programs.

TABLE 1. SUMMARY OF THE FIR ROUTINES

+---------------------------+-----------------------------------+

|	Memory (Bytes)      |     Run-Time Required             |

+---------------------------+-----------------------------------+

|	 Filter    State    |  No. of	      No. of            |

Routine |  Code	 Coeff.	  Variable  | Instructions   Wait States        |

--------+-------+-------+-----------+--------------+--------------------+

fir	|   60	   4N     4(N\(mi1) |   11 + 2N       2,(4N\(mi5)       |

--------+---------------------------+-----------------------------------+

firF	| 36+4N	   4N     4(N\(mi1) |   11 + N        2,(4N\(mi6)       |

--------+---------------------------+-----------------------------------+

fir5	|   88	   4N     4(N\(mi1) |   12 + 1.2N     2,(4N\(mi4\(miM)  |

--------+---------------------------+-----------------------------------+

TABLE 2. SUMMARY OF THE IIR ROUTINES

+---------------------------+-----------------------------------+

|	Memory (Bytes)      |     Run-Time Required             |

+---------------------------+-----------------------------------+

|	 Filter    State    |  No. of	      No. of            |

Routine |  Code	 Coeff.	  Variable  | Instructions   Wait States        |

--------+-------+-------+-----------+--------------+--------------------+

iir     |   64    4+16N      8N     |   13 + 5N       N+2,(13N+1)       |

--------+---------------------------+-----------------------------------+

iirN    | 36+16N  4+16N      8N     |   11 + 4N       2N+1,(14N\(mi1)   |

--------+---------------------------+-----------------------------------+

iirt    |   64     20N       8N     |   12 + 6N       N+1,(9N\(mi1)     |

--------+---------------------------+-----------------------------------+

iirtN   | 32+20N   20N       8N     |   10 + 5N       2N+1,(10N\(mi1)   |

--------+---------------------------+-----------------------------------+

To facilitate program development using the DSP32 Digital Signal

Processor, the WE DSP32-AL Application Software Library [11] contains

many other signal processing routines in addition to the filtering

routines described in this application note. The DSP32-G08-AG [12]

device's on-chip ROM contains the routines fir5 and iir, as well as

twelve other routines from the DSP32-AL Application Software Library.

In addition, the DSP32-DS DSP Development System can be used as a

prototyping station. With a PC-based filter design program, these

routines, and the DSP32-DS DSP Development System, they can turn

the tedious and time-consuming job of hardware prototyping digital

FIR/IIR filters into an instant (i.e., matter of seconds) reality.

REFERENCES

~~~~~~~~~~~

[1] WE(R) DSP32 Digital Signal Processor Information Manual, December 1986,

AT&T Technologies, 555 Union Boulevard, Allentown, PA 18103

[2] Oppenheim, A. L., and Schafer, R. W., "Digital Signal Processing,"

Prentice-Hall, 1975.

[3] Jackson, L. B., "Digital Filters and Signal Processing,"

Kluwer Academic Publishers, 1986.

[4] Szentirmai, G., "FILSYN: A General Purpose Filter Synthesis Program,"

"Proc. IEEE," Vol 65, October 1977, pp. 1443-1458.

[5] "Programs for Digital Signal Processing," IEEE Press, 1979.

[6] DFDP (Digital Filter Design Package), Atlanta Signal Processors

Incorporated, 770 Spring Street, Suite 208, Atlanta, GA 30332, 1985.

[7] DISPRO v1.5, "Digital Signal Processing Software for the Design

and Evaluation of Digital Filters," Signix Corporation.

[8] "Filter Design and Analysis Program," Momentum Data Systems, Inc.,

1666 Newport Boulevard, #~115, Costa Mesa, CA 92627.

[9] WE(R) DSP32-DS Digital Signal Processor Development System User Manual,

Dec. 1986, AT&T Technologies, 555 Union Boulevard, Allentown, PA 18103

[10] WE(R) DSP32-SL Support Software Library User Manual, March 1987,

AT&T Technologies, 555 Union Boulevard, Allentown, PA 18103,

[11] WE(R) DSP32-AL Application Software Library Reference Manual,

Release 1.0, April 1987.

[12] WE(R) DSP32-G08-AG Digital Signal Processor Information Manual,

July 1988, AT&T Technologies, 555 Union Blvd., Allentown, PA 18103,

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=