AT&T Technologies                                    Document No. AP88-18DMOS

November 1988

NOISE GENERATION ROUTINES

USING THE

WE(R) DSP16 DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

noise16.doc   This document.

random1.s     Program Segment 1. Linear Congruential method of RNG^.

random2.s     Program Segment 2. Linear Congruential method of RNG.

Optimized for modulus of power of two.

random3.s     Program Segment 3. RNG using built-in generator.

gaussn4.s     Program Segment 4. Using table look-up to generate GDD^.

gaussn5.s     Program Segment 5. Using table look-up to generate GDD.

With interpolation on result.

iir6.s        Program Segment 6. IIR filter for random number sequence.

mktab23.c     Appendix 1. Linear congruential method of RNG.

mktab4.c      Appendix 2. Table generation for Program Segment 4.

mktab5.c      Appendix 3. Table generation for Program Segment 5.

errf.c        Appendix 4. Gaussian or error function.

^RNG == Random number generation

^GDD == Gaussian distributed deviates

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1988 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

CONTENTS

~~~~~~~~~

Introduction			(included below)

Random Number Generation

Noise Generation With Specific

Probability Function

Noise With Specific Spectral

Density Characteristics

Conclusion			(included below)

Summary of DSP16 Programs	(included below)

References			(included below)

Appendices			(separate files)

INTRODUCTION

~~~~~~~~~~~~~

Digital signal processors (DSPs) have the capability to execute sequences of

instructions with precision, accuracy, and repeatability. However, there are

times when controlled amounts of variability are required or desired when

performing calculations. Variability may be inserted by using random numbers.

When sequences of random numbers are used as signals, they are described as

noise sources.  Applications for noise sources include:

1.  Equalization of channels for modems.

2.  Dithering of speech and video signals with deterministic noise

prior to quantization in order to increase the preceived quality

of the receive signal [2].

3.  Replacing gaps in speech signals where silence has been removed

for transmission of storage efficiences, allowing more natural

sound when reconstructed with background noise at the same level

as the orginal speech [3].

4.  Improving performance of signal processing algorithms by adding

low-level noise to the signal being analyzed [2].

5.  Applications requiring random numbers such as random tree search,

random codebook search, and Monte-Carlo analysis.

There are two basic methods of generating random number sequences. The first

is to measure some truly random quantity such as the time between counts of

a Geiger Counter, the voltage across a large resistance, or thhe phase

difference between two free-running oscillators. Although these examples of

measurement will produce random number sequences, they suffer from several

deficiencies:

1. Portability. These methods may not be practical for all situations.

2. Repeatability between systems. It would be difficult to guarantee that

two instances of such physically-based random number sources produce

sequences with the same characteristics.

3. Repeatability of time. There are times when the same random number

sequence is required to evaluate modifications made to an algorithm.

These deficiencies are eliminated if the random sequences are generated by a

deterministic processor (the second general method of generating random number

sequences). However, a new problem arises, namely insuring that the sequence

generated is truly random.

This application note will describe noise generation using the DSP16 Digital

Signal Processor.  Following this section, section two describes several

random number generation algorithms; section three describes methods of

processing a random number sequence to produce different probability

distributions; and section four describes methods of modifying the spectrum

of the noise signal generated from a random number generator.  DSP16 code

is given and described for all of these techniques.

CONCLUSION

~~~~~~~~~~~

The generation of random numbers and noise is often used in speech and signal

processing. This application note has shown that the DSP16 can be used to

generate random numbers and noise efficiently for real-time applications.

SUMMARY OF DSP16 PROGRAMS

~~~~~~~~~~~~~~~~~~~~~~~~~~

Table 6 shows a summary of the DSP16 program segments described in this

application note.

TABLE 6.  Summary of DSP16 Programs

--------------------------------------------------------------------

Program                                ROM        Cycles (not incl.

Segment      Description          (ROM + tables)    called routines)

--------------------------------------------------------------------

1      Linear Congruential #1        54              65

2      Linear Congruential #2        31              37

3      Built-in Random Bit            7              38

Generation

4      Gaussian Deviates #1          45              39

(29 + 16)

5      Gaussian Deviates #2          69              70

(52 + 17)

6      IIR filter                    20              25

--------------------------------------------------------------------

REFERENCES

~~~~~~~~~~~

[1] Widrow, B., and Stearns, S., "Adaptive Signal Processing",

(Englewood Cliffs, New Jersey: Prentice Hall, 1985).

[2] Jaynat, N.S., and Noll, P., "Digital Coding of Waveforms",

(Englewood Cliffs, New Jersey: Prentice Hall, 1984).

[3] Feher, K., "Advanced Digital Communications",

(Englewood Cliffs, New Jersey: Prentice Hall, 1987).

[4] Knuth, D.E., "The Art of Computer Programming",

(Reading, Massachusetts: Addison Wesley, 1981).

[5] Press, W.H., Flannery, B.P., Teukolsky, S.A., and Vetterling, W.T.,

"Numerical Recipies", (Cambridge: Cambridge University Press, 1986).

[6] Papoulis, A., "Probability, Random Variables and Stochastic Processes",

(New York: McGraw Hill, 1965).

[7] Hillier, F.S., Lieberman, G.J., "Introduction to Operations Research",

(San Francisco: Holden-Day, 1968).

[8] Rabiner, L.R., Gold, B., "Theory and Applications of Digital Signal

Processing", (Englewood Cliffs, New Jersey: Prentice Hall, 1975).

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=