AT&T Technologies                                    Document No. AP88-20DMOS

March, 1989

INTERFACING EXTERNAL RAM

TO THE

WE(R) DSP16 FAMILY OF DIGITAL SIGNAL PROCESSORS

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

exm16.doc:	This document.

exm16.s:	Program Listing.  Sample Program on page 8-9.

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1989 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

CONTENTS

~~~~~~~~~

Introduction				(included below)

Parallel I/O Control Register		(included below)

External Device Access Time

Active/Passive Mode

Status/Control Bit Mode

Parallel I/O Ports			(included below)

Addressing External RAM

With Autoincrement/Autodecrement

Circuit Description

Circuit Operation

Addressing External RAM

Without Autoincrement/Autodecrement

Sample Program				(in separate file)

INTRODUCTION

~~~~~~~~~~~~~

This application note provides a guide to interfacing external RAM to

the WE DSP16 or DSP16A Digital Signal Processor.  Two circuits are shown

in this note.  Which is used depends on whether address transmittal time

is crucial to the application.

The DSP16 and DSP16A Digital Signal Processors have an on-chip static RAM

space of 512 words and 2048 words, respectfully.  Variable operands, such

as adaptive filter coefficients, state variables, intermediate results,

and I/O data are stored in this on-chip RAM.  If the on-chip RAM space

is insufficient, external RAM is accessible through the parallel I/O

interface (PIO) unit.  The circuit shown in Figure 1 allows for both

random and sequential accessing of up to 64 Kwords of external RAM.

However, a higher data transfer rate is possible when RAM is accessed

sequentially.

PARALLEL I/O CONTROL REGISTER

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The parallel I/O control (PIO) register is a 16-bit register used to

provide I/O and interrupt status and to configure the following features

of the parallel I/O:

* External device access time

* Interrupt mask

* Active/passive mode

* Status/control (S/C) bit mode

PARALLEL I/O PORTS

~~~~~~~~~~~~~~~~~~~

Although there is only one physical PIO port, there are two logical ports,

pdx0 and pdx1.  These two logical ports are distinguished by the state of

the peripheral select line (PSEL).  A 1 on PSEL means that the logical port

pdx1 was the last to be referenced; conversely, a 0 on PSEL means that pdx0

was the last to be referenced.  Because the DSP16/DSP16A does not have an

address bus for its external memory, logical port pdx1 is usually used to

transfer the address of the data if an address is required.  Thus a write

to pdx1 causes PSEL to become or remain a 1, indicating that data is being

transmitted on the physical port must be interpreted as an address.

Following the write to pdx1 is another write to pdx0, which pulls PSEL to

a 0, indicating that data is being transmitted.  Thus, an access to external

memory implies two PIO transactions for each memory transaction (a write to

pdx1 for the address and a read or write to pdx0 for data).

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=