AT&T Technologies                                  Document No. AP89-002DMOS

April, 1989

DUAL-TONE MULTIFREQUENCY RECEIVER

USING THE

WE(R) DSP16 DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

s16dtmf1.s: Variation of Program Listing 1.  Page 10.

Sets sioc=0x43  for  8 bit I/O, passive clocks, MSB first

When a valid digit is detected, it is written to location 0

of PRAM.  All following detected valid digits are appended

to this location in PRAM.

s16dtmf2.s: Variation of Program Listing 1.  Page 10.

This version is for simulation only.

Sets sioc=0x7f  for  8 bit I/O, active clocks, MSB first

When a valid digit is detected, it is written to the PIO.

s16dtmf3.s: Program Listing 1.  DFT Based DTMF Receiver;  Page 10.

Sets sioc=0x43  for  8 bit I/O, passive clocks, MSB first

When a valid digit is detected, it is written to the PIO.

s16dtmf1.ex: \

s16dtmf2.ex:  > exec files for above programs.

s16dtmf3.ex: /

m16dtmf.s:   Program Listing 2.  16 DFT Based DTMF Receivers;  Page 21.

sdx.in:      Test input, 2800 samples.

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The remainder of this file gives further notes about the programs

and is designed to take you from simulating 1 DTMF receiver to

testing 16 DTMF receivers in real time.

PROGRAM:  s16dtmf2.s

----------------------------------------------------------------------------

s16dtmf2.s implements 1 DTMF receiver.  When a valid digit is detected, it

is written to the PIO.  Since the clocks are set up as active, it will not

run on the Development System.  It is setup this way so that it may be tested

with the simulator only.

To test s16dtmf2.s in the simulator you need to create an external

file of 8 bit mu-law samples of touchtone signals.  To get you started,

the files s16dtmf2.ex and sdx.in will take you through a sample simulation.

sdx.in contains 4 50msec touchtone digits (1,5,9,d) each separated by

50msec of silence.  s16dtmf2.ex contains all of the env16 commands needed

to run the program.  To run this sample simulation you must have s16dtmf2.s,

s16dtmf2.ex, and sdx.in in your present directory and execute the following

command:

env16 -e s16dtmf2.s

When using the simulator in env16, anything written to the PIO gets written

into a file called pdx.out. So after env16 has completed the simulation and

exited after 338455 machine cycles, view the file pdx.out. It should contain

the following data:

0 0x0001

0 0x0005

0 0x0009

0 0x000d

The first column means that pdx0 was written to and the second column is

the detected digit.

PROGRAM:  s16dtmf3.s

-------------------------------------------------------------------------------

s16dtmf3.s is the same as s16dtmf2.s except for the clocks which are setup

as passive.  Therefore s16dtmf3.s can be executed on the Development System

and run in real-time by executing:

env16 -e s16dtmf3.s

To read the valid digits there must be external hardware connected to the

PIO or the Development System can be used as an emulation tool and plugged

into your actual hardware instead of the DSP16 chip.

PROGRAM:  s16dtmf1.s

-------------------------------------------------------------------------------

If you want to test a single receiver in real-time with no external hardware,

download the file s16dtmf1.s into the Development System by executing:

env16 -e s16dtmf1.s

Every time a valid digit is detected it is written to PRAM starting at location

zero.  To read PRAM you must hit CNTRL BREAK to reset the DSP.  This technique

gives you the flexibility of being able to run the program for any length of

time versus running till a breakpoint is hit (NOT RECOMMENDED!!!!!).  After

the DSP16 is reset view PRAM by typing:

sim:PRAM[0:n]

where n is any value greater than 0.

PROGRAM:  m16dtmf.s

-------------------------------------------------------------------------------

m16dtmf.s implements 16 DFT based DTMF receivers on the

AT&T WE (R) DSP16 operating at a 55nsec instruction rate and uses

only internal memory.

The DSP16 receives sampled data in mulaw format through the serial

port which is interrupt driven with the software interrupt IBF(input

buffer full).  Every 125usec a hardware interrupt occurs to

synchronize the software.

When a digit has been validated, it is written to the PIO.  The lower

byte of pdx0 contains the channel number in the upper 4 bits, and the

detected digit in the lower 4 bits.

All clocks are passive.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=