AT&T Technologies                                    Document No. AP89-009DMOS

June 1990

DIGITAL ECHO CANCELER

AND

COMPLEX ADAPTIVE EQUALIZERS

USING THE

WE(R) DSP16/DSP16A DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

eko-eql.doc    This document.

echo-sp.s      Program listing 1. Echo Canceler - Single Precision

echo-dp.s      Program listing 2. Echo Canceler - Double Precision

equal-l.s      Program listing 3. Complex Linear Equalizer

equal-df.s     Program listing 4. Complex Decision Feedback Equalizer

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1989 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

CONTENTS

~~~~~~~~~

Introduction          (included below)

Digital Echo Canceler Implementation        (included below)

Digital Echo Canceler for Baseband Data Transmission

DSP16/DSP16A Implementation of the Echo Canceler

Simulation Results of the Echo Cancelers

Conclusion        (included below)

References        (included below)

Program Listing 1. Echo Canceler - Single-Precision

Program Listing 2. Echo Canceler - Double-Precision

Complex Adaptive Equalizers        (included below)

System Description and Algorithm

DSP16/DSP16A Implementation of Complex Adaptive Equalizers

Testing of the Equalizer

Conclusions       (included below)

References        (included below)

Program Listing 3. Complex Linear Equalizer

Program Listing 4. Complex Decision Feedback Equalizer

INTRODUCTION

~~~~~~~~~~~~~

Echo cancellation techniques have been widely used in baseband digital

transmission systems to provide full duplex data traffic over a two-wire

network. The implementation of an echo canceler using the WE DSP16/DSP16A

Digital Signal Processor (DSP) is given in the first section of this

application note.  The DSP16/DSP16A is a general-purpose, 16-bit DSP.

The echo canceler is based on an adaptive transversal filter using the

least mean square (LMS) algorithm to update the filter coefficients.

Two examples are given: one employs single-precision filter coefficients

as well as single-precision algorithm calculations; the other uses

double-precision arithmetic, except that the input and output data are

represented as single-precision words.  The trade-offs between the two

systems are the data rate and echo rejection.  The double-precision

implementation can offer better echo rejection while the single-precision

design allows a system with a higher data rate.

Complex adaptive equalizers are a widely used building block in digital

communications systems.  Their function is to equalize channel distortions

in order to reduce intersymbol interferences (ISI) at the receiver.

The second section of this application note describes complex adaptive

equalizer implementations for both linear equalizers (LE) and decision

feedback equalizers (DFE) using the DSP16/DSP16A.  The LMS adaptation

algorithm is employed to update the equalizer coefficients.  Each equalizer

is tested by applying a 16-point quadrature amplitude modulated (16-QAM)

signal transmitted through a long-haul voiceband telephone channel.  For an

8-tap LE, the DSP16/DSP16A program takes a maximum of 239 cycles to process

each input data symbol.  For a DFE consisting of a 3-tap forward equalizer

and a 5-tap feedback equalizer, the program execution takes 265 cycles.

Using a 55 ns DSP16/DSP16A processor, the throughputs for the linear and

decision feedback equalizers are under 14 us and 15 us, respectively.

DIGITAL ECHO CANCELER IMPLEMENTATION

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

A two-wire, full duplex baseband data transmission system usually relies

on an echo canceler to decouple the outgoing transmitted signal and the

incoming received signal.  An example of this system is the integrated

service digital network's (ISDN) U interface which supports 160 Kbit/s

(80K baud using 2B1Q - two binary to one quaternary - line code) data

rate on a subscriber loop over distances up to 5 Km [1]. The application

of the echo canceler in systems of various data rates and distance

coverages has also been reported [2,3].  In most systems, the worst case

signal attenuation of 40\(em50 dB can be expected.  Therefore, in order

to achieve a signal-to-echo ratio of 20 dB as required for low bit error

rate (BER) transmission, the echo canceler has to provide 60-70 dB echo

rejection.

The echo canceler can be implemented as an adaptive transversal filter

which automatically adapts its coefficients to the impulse response

of the echo path.

If the echo path is a linear system and the transversal filter contains

sufficient taps to cover the entire period of the echo's impulse response,

the echo replica generated by the transversal filter can completely cancel

out the echo signal from the received signal.

However, due to the finite precision of the DSP and the

tap misadjustments inherent to the adaptation algorithm, a small residual

echo always exists.

The LMS algorithm [4] is the most popular algorithm used in adaptive

transversal filters. The computational requirements (measured by the

multiplication/addition operation) is 2N+1 operations per data sample,

where N is the number of taps. The implementation of the LMS adaptive

filter using a fixed-point processor introduces two major sources of

error: one is from the quantization error in the filter coefficients,

and the other arises when the tap updates become smaller than the least

significant digit of the processor.  This causes the adaptation process

to stall.

In this section, a 48-tap echo canceler implemented using both single-

precision and double-precision arithmetic is described.  Both

implementations are tested using a simulated two-wire circuit which

consists of a 22-gauge 10 Km loop, the termination network, and the

transmit and receive filters.  The results are compared with those

obtained from using floating point arithmetic.

CONCLUSION   (Echo Cancellor)

~~~~~~~~~~~

The implementation of two digital echo cancelers for baseband data

transmission using the DSP16/DSP16A DSP is described in this application

note. The single-precision echo canceler has a throughput advantage and

occupies less memory space than its double-precision counterpart.

However, the double-precision echo canceler offers much greater degree

of cancellation and is limited only by the 16-bit quantization noise.

The speed and flexibility of the DSP16/DSP16A device lends itself to the

implementation of echo cancelers in two-wire full duplex baseband data

transmission systems.

REFERENCES   (Echo Cancellor)

~~~~~~~~~~~

1. O. Agazzi, "Cable Transmission Techniques for ISDN: A Tutorial",

Proceedings of the II Congresso da Sociedade Brasileira de

Microeletronica, July, 1987, Sao Paulo, Brazil, pp. 623-632.

2. T. Alvestad, T. J.-C. Eriksen, "Echo Canceler for Two-Wire Data Modems",

Electrical Communication, Vol. 59, No. 3, 1985, pp. 333-337.

3. N.A.M. Verhoeckx et al. "Digital Echo Cancellation for Baseband Data

Transmission", IEEE Trans. Acoust., Speech, Signal Processing,

Vol. ASSP-27, pp. 768-781, Dec. 1979.

4. B. Widrow, S. D. Sterns, Adaptive Signal Processing, Prentice-Hall,

Englewood Cliffs, New Jersey, 1985.

COMPLEX ADAPTIVE EQUALIZERS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Transmitting high bit-rate data successfully through a switched telephone

network, microwave or satellite link, etc., often requires adaptive

equalization to remove excessive channel distortions (i.e., amplitude and

phase distortion or multipath fading distortion).  The adaptive equalizer

is usually implemented using transversal filters with certain algorithms

controlling the update of the coefficients [1].  The two most popular

structures used in adaptive equalization are the LE and DFE.  The LE,

shown in Figures 5 and 6, derives its output by linearly combining the

past received signals weighted by the coefficients.  The DFE, shown in

Figures 7 and 8, is comprised of both a forward equalizer and a feedback

equalizer. The output of the DFE is the sum of the two equalizers. The DFE

is found to be particularly effective when the channel is highly distorted.

In addition, it offers the advantages of being less sensitive to symbol

timing accuracy and having less noise enhancement problems, as compared

to linear equalizers.

In this section, the implementations of an 8-tap complex linear equalizer

and a DFE of 3 forward taps and 5 feedback taps using the DSP16/DSP16A

processor are presented. Included is a description of the system blocks

and the algorithm, the DSP16/DSP16A programs, and the testing results of

the equalization of a 16-QAM signal sent through a dispersive voiceband

telephone channel.

CONCLUSION     (Complex Adaptive Equalizers)

~~~~~~~~~~~

Implementations of an 8-tap complex linear equalizer and a decision

feedback equalizer of 3 forward taps and 5 feedback taps using the

DSP16/DSP16A DSP are given in this application note.  Using a 55 ns

processor, the throughputs of the LE and DFE are under 14 us and 15 us,

respectively.  The misadjustments of both adaptive equalizers at steady

state are very low even though the full dynamic range

( -2 <= X <= 2-2**(-14) of Q14 format; signal range is -0.4 < X < 0.4 )

is not utilized.  This low level of misadjustment is achieved by taking

advantage of the two 36-bit accumulators and by using rounding instead

of truncation (default operation) prior to the transfer of data from an

accumulator to data memory or other 16-bit registers. The execution of

the LMS complex adaptive filter alone takes 17 cycles per tap including

rounding the new coefficient and writing it to the data memory.

REFERENCES      (Complex Adaptive Equalizers)

~~~~~~~~~~~

1. S. U. H. Qureshi, "Adaptive Equalization"  Proceedings IEEE,

Vol. 73, No.9, Sept. 1985, pp. 1349-1387.

2. B. Widrow, S. D. Sterns, Adaptive Signal Processing, Prentice-Hall,

Englewood Cliffs, New Jersey, 1985.

3. F. P. Duffy, T. W. Thatcher, "Analog Transmission Performance on the

Switched Telecommunications Network" BSTJ, Vol. 50, No. 4. Apr. 1971,

pp. 1311-1346.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=