AT&T Technologies                                    Document No. AP89-012DMOS

July 1990

INTERFACING THE WE(R) DSP32C

DIGITAL SIGNAL PROCESSOR

TO EXTERNAL MEMEORY

NOTICE

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1990 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

exm32.doc	This document.

eboot.asm	Program Listing 1. EPROM Boot Program (segment)

fig6.pld	Figure 6.  PAL Listing, pg 16.

fig10.pld	Figure 10. PAL Listing, pg 23.

d1gold.abl	Program Listing 2. Memory Arbitration and State Generation PAL.

d2gold.abl	Program Listing 3. RAS and CAS Generation PAL.

d3gold.abl	Program Listing 4. DRAM Timing Generation PAL.

d4gold.abl	Program Listing 5. DRAM Address Strobing PAL.

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

CONTENTS

~~~~~~~~~

Introduction				(included below)

The External Memory Interface		(included below)

Signal Description			(included below)

Signal Loading

Interfacing the DSP32C to External Memory

Single-Bank Interface to ATT41298 Static RAM

Two-Bank Interface to CY7C185 Static RAM

Four-Bank Interface to ATT41256 Static RAM

Eight-Bank Interface to ATT41298

Interface to M5M27C102K EPROM

Interface to CY7C291A EPROM

Interface to M5M5256P Static RAM

Interface to ATT41258 RAM Without OE

Interface to IDT7132 Dual-Ported Static RAM

Program Listing 1. EPROM Boot Program - DSP in Memory Mode 2

Interface to M5M44258AL Dynamic RAM

PAL Equations For Interface 2.9		(separate files)

Program Listing 2. Memory Arbitration and State Generation PAL

Program Listing 3. RAS and CAS Generation PAL

Program Listing 4. DRAM Timing Generation PAL

Program Listing 5. DRAM Address Strobing PAL

INTRODUCTION

~~~~~~~~~~~~~

This application note is a guide to interfacing external memory

to the WE DSP32C Digital Signal Processor by using the DSP32C's

external memory interface (EMI) port.  Ten circuits are shown

in this document.  The type of memory used dictates the circuit

required.  The most available commercial memory devices are used

for each described circuit.

THE EXTERNAL MEMORY INTERFACE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The DSP32C device is available in two different memory configurations.

The first is a ROM-less device with 1536 words of RAM, and the second

has 1024 words of RAM and 4096 words of ROM.  Additionally, up to 16

Mbytes of external memory can be directly addressed by the external

memory interface that supports wait states and bus arbitration.  All

memory can be accessed as 8-, 16- or 32-bit data, with the 32-bit data

being accessed at the same speed as 8- or 16-bit data.

All external memory accesses are transparent to the user.  The on-chip

address and data latches and external memory control pins allow for a

zero-chip interface to standard byte-wide memory.

The DSP32C address space is composed of four physical memories: external

memory, RAM0, RAM1, and RAM2.  A physical memory may be accessed twice

per instruction cycle (80 ns).  Thus, the external memory cycle time is

40 ns (assuming 80 ns instruction cycle and zero-wait-state mode).  To

better understand this 40 ns memory cycle time, each DSP32C process

consists of at least four states (CKI clock cycle).  Additional states

may be added for memory bank conflicts, DMA operations, interrupts and

external-memory-bus wait states.

The four normal states on the address and data buses are:

1. Instruction Fetch

2. Delayed DAU (DEST) write

3. DAU (XSRC) or CAU read

4. DAU (YSRC) read or CAU write.

Recall that the DSP32C address space is composed of four physical memories:

(1) external memory, (2) RAM0, (3) RAM1, and (4) RAM2 or ROM.  Memory

accesses can be made without regard to which memory is being accessed.

However, to achieve maximum throughput, memory access must be interleaved

between physical memories.  As one memory is being accessed, the other

memory bank is being addressed.  In figure 1, the instruction fetch and

DAU/CAU read are in one physical memory and the delayed DAU write and

DAU read or CAU write are in another.  Since each physical memory may be

accessed twice per instruction cycle, the external memory may be accessed

twice per instruction cycle.  This results in a 40 ns memory cycle time.

The external memory interface read transaction is used to convey data

from the external system to the DSP32C. The write transaction is used

to convey data from the DSP32C to the external system.

Read transactions may be initiated by instruction fetches, register

load operations, operand fetches, or DMA fetches. Write transactions

may be initiated by register store operations, DAU writes, or DMA

operations.  The data in these transactions are transmitted as bytes

(8-bits), integers (16-bit), or words (32-bit). Instruction fetches

are always 32 bits.  Operands that are 24-bits are fetched and stored

by using 32-bit transactions.

SIGNAL DESCRIPTION

~~~~~~~~~~~~~~~~~~~

The external memory interface port is a general-purpose parallel

port that can be used for interfacing to static and dynamic RAM

or for interfacing to memory mapped devices other then memory. The

DSP32C memory interface consists of the following signals. For

a complete description of these signal functions, refer to the

WE(R) DSP32C Information Manual):

* AB00-AB21   An address for each memory access is issued on the

external address bus. This applies to both internal and external

access. These signal are in an undefined state between accesses.

* MSN0-MSN3   The byte-select lines indicate which byte within a word

are being accessed. These lines are active only during external

accesses. The lines are the decoded output of the least significant

bits of the internal address bus. The timing of these signal is the

same as AB00-AB21.  These signals are in an undefined state between

accesses.

* ASN   This signal indicates that a valid external address is on the

address bus. This signal is active only during external access and

is always high between external accesses.

* DB00-DB31   The external data bus can transfer 8, 16, 24 and 32-bit

data. The only time the data bus is not in high impedance is during

an external write.

* DSN   This signal indicates that data may be placed on the data bus

(read cycle) or data is valid on the bus (write cycle). This signal is

active only during external access and is always high between accesses.

* CYCLEIN   This signal indicates the start of an external memory cycle;

it is active for one clock cycle. It is only active during external

access, and is always high between accesses. The signal is not extended

with the addition of memory wait states.

* RWN   This signal indicates that the external access is a write (low)

or read (high). It is only low during external writes and stays low

during back-to-back external writes.

* MGN   This signal has two functions. If PCW[8] = 0, MGM indicates

that data may be placed on the data bus. This signal is active only

during external reads, and is always high between accesses.

If PCW[8] = 1, MGM is used to support bus arbitration. When low,

it indicates that the DSP32C has an external access pending but

does not have ownership of the bus.

* MWN   This signal indicates that data on the bus is valid. This

signal is active only during external writes and is always high

between accesses.

* SRDYN   This signal is an input to the DSP32C device indicating when

to terminate an external memory transaction. The signal can be used

in an asynchronous or synchronous manner. Conformance to the set-up

and hold times of SRDYN with respect to CKO will guarantee termination

of the transaction 2.5 clock cycles later.  If the signal is not used

it should be tied low, since after reset, the DSP32C device starts up

in the synchronous-wait-state mode.

* CKO   This is a free-running clock with the same frequency as the

system clock. Many of the memory timing specifications refer to

this clock.

* BREQN   This signal places the external memory interface in a high

impedance state. When BREQN is asserted, any current external memory

transaction is completed. The address, data, CYCLEIN, ASN, DSN, RWN,

MGN (PCW[8] = 0), MWN and MSN0-MSN3 signals are then placed in a high

impedance state until BREQN is brought high.  While this signal is

active, the DSP32C will continue to execute out of internal RAM.

If the BREQN feature is not used, the signal must be tied high.

* EAPN   When low, this signal indicates that the DSP32C has an

external access pending, but does not have ownership of the bus.

The logical value of PCW[9] is ORed with an internal signal

that indicates that an external access is pending

(i.e., if PCW[9] = 1, EAPN = 0).

* BRACKN   This signal indicates that the external memory interface

is in high-impeadance as a result of BREQN being active.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=