AT&T Technologies                                    Document No. AP89-015DMOS

November 1989

INTERFACING THE WE(R) DSP16 DIGITAL SIGNAL

PROCESSOR TO MULTIPLE

A/D  AND  D/A  CONVERTERS

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

ad-da16.doc   This document.

pass4.s       Program Listing 1. Example 1: Pass data through all four channels

robin4.s      Program Listing 2. Example 2: Round-Robin of all four channels

avrg4.s       Program Listing 3. Example 3: Average all four channels

csport.s      Program Listing 4. Example 4: Control/status port use

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1989 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

CONTENTS

~~~~~~~~~

Introduction                    (included below)

Review of the DSP16             (included below)

Parallel I/O Port           (included below)

PIOC Register               (included below)

ROM Bus                     (included below)

Methods of Parallel Interface   (included below)

Dual-Cycle, Parallel I/O Bus

Single-Cycle, Parallel I/O Bus

ROM Bus Extension

Implementation

DSP16 Memory Interface

DSP16 Timing Generation

Decode Logic

Analog Input

S/H and A/D Converter

Analog Output

Data Formats

Noise Considerations

Other Considerations

Programming Examples

Example 1, 2, 3, 4          (included below)

Summary                     (included below)

Conclusion

Program Listings		(in separate files)

Schematics

INTRODUCTION

~~~~~~~~~~~~~

A common use of the WE DSP16 Digital Signal Processor is for processing

audio signals.  These signals range from narrow-band speech on a public

telephone network, to wide-band audio for high-fidelity broadcasting.

Prior to any processing by the DSP16 device the audio signals must be

converted from an analog signal to a digital signal. The resulting output

must then be converted back to an analog signal. This conversion is

accomplished by using analog-to-digital (A/D) and digital-to-analog (D/A)

converters.

Voice signals used on the telephone network are converted by standard

u-law/A-law codecs, such as the AT&T T7500 PCM Codec. The analog waveform

of the signal to be converted is sampled at an 8 kHz rate, using an 8-bit,

non-linear encoding scheme. The result is an aggregate bit rate of

64 kbits/s and provides for toll-quality voice encoding.

For high-quality broadcasting or encoding of music, the conversion must

be more precise than that used for voice. Generally, the sample rate

needs to be greater than twice the highest analog frequency of interest.

Audible signals range up to 20 kHz, requiring at least 40 kHz sampling

rate.  Using a linear encoding scheme and requiring that each sample

have 14-16 bits makes a 640 kbits/s broadcast-quality encoding.

The A/D and D/A converters that meet these more stringent specifications

are specialized and more complex than the standard telecommunications

codecs. The application note discusses the use of two of these devices

with the DSP16 device.

The remainder of this application note consists of four parts. The first

part reviews the features of the DSP16 device that are used to interface

to the converters.

The second part describes three alternative approaches to incorporating

the converters with the DSP16 device.  This include block diagrams and

program segments.  These techniques can be easily extended to interface

with other types of devices.

The third part describes the detailed implementation of one of these

methods using the 16-bit Burr Brown PCM78 analog-to-digital converter,

the 16-bit Burr Brown PCM53 digital-to-analog converter, and the

Signetics TDA1535N sample and hold.  This circuit is operational.

The final part describes software considerations and presents

several examples that exercise the hardware. Design considerations

and performance are also discussed.

REVIEW OF THE DSP16

~~~~~~~~~~~~~~~~~~~~

The DSP16 device supports both a parallel and a serial I/O port.

Demonstrated in this application note, are various uses of the parallel

port of the DSP16, which allows for a simpler interface circuit as well

as easy expansion by using multiple devices.

Parallel I/O (PIO) Port

~~~~~~~~~~~~~~~~~~~~~~~~

The DSP16 device has the capability of interfacing to I/O devices via its

parallel I/O port.  The PIO signals consists of one 16-bit, parallel I/O

bus (PDB00-PDB15), a port-selection pin (PSEL), and port read (PIDS) and

port write (PODS) pins.  The parallel I/O interface is logically defined

as two bi-directional parallel ports, referred to as pdx0 and pdx1. These

ports are differentiated in hardware by the state of the PSEL pin. Ports

pdx0 and pdx1 are accessed by the data move group of instructions of the

DSP16 device.  When writing to the pdx0 or pdx1, the PODS pin pulses low;

when reading pdx0 or pdx1, the PIDS pin pulses low. The pin PODS, PIDS and

PSEL are driven by the DSP16 if they are selected to be active by the

setting of the PIOC register.  The WE(R) DSP16 Digital Signal Processor

Data Sheet dicusses the timing of these signals.

PIOC Register

~~~~~~~~~~~~~~

The parallel I/O Control (PIOC) controls the PIO bus and the serial bus

and also controls the masking of various interrupts that the DSP16 device

may receive.  Through the PIOC, the PIDS and PODS signals can be configured

in active mode (i.e., driven by the DSP16 device) or passive mode (driven

by an external signal).  In addition, when the PIDS and PODS signals are

configured in the active mode, the STROBE bits specify the duration of these

signals.  If the PIDS and PODS signals are passive, the DSP16 device is

interrupted by the INTERRUPT bits of the PIOC whenever a read or write

is performed on the parallel port.  Status bits of the serial and parallel

ports are also found in the PIOC register.  These bits are read by the

DSP16 device to perform polled I/O with the ports.

ROM Bus

~~~~~~~~

The ROM data bus (RB00-RB15) and ROM address bus (AB00-AB15) are used by

the DSP16 device to execute programs from external memory.  A data

strobe signal is not generated when the DSP16 reads the program memory.

The address is asserted and the data is read by the DSP16.  The DSP16

device running at 36.36 HHz requires ROM memory with 35 ns access time.

The DSP16 running at 60.6 MHz requires memories with a 15 ns access time.

The ROM bus is also used to control access to external peripherals.

This is discussed in the ROM Bus Extension section.

METHODS OF PARALLEL INTERFACE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Three different techniques for accessing multiple external peripherals

are presented.  The first uses the parallel I/O ports for both addressing

the peripherals and accessing the peripheral's data.  The second method

uses extra hardware to simplify the addressing of the peripherals on the

parallel I/O bus.  The third method uses the ROM bus to address the

peripherals and the parallel I/O bus to access the data.  Each of these

methods has its advantages and disadvantages, which are discussed next.

PROGRAMMING EXAMPLES

~~~~~~~~~~~~~~~~~~~~~

Example 1

~~~~~~~~~~

The program shown in Program Listing 1 simply passes data straight

through all four channels. At each interrupt, the input data of each

A/D is read and then written to the D/A of the same channel. In this

example, latent reads are handled by reading each A/D twice after

selecting the channel. This method increases the time needed to

service each channel, but is easy to understand and implement.

Example 2

~~~~~~~~~~

The program shown in Program Listing 2 reads every A/D converter and

writes to the subsequent D/A converter. Therefore, the analog waveform

digitized by D/A 0 is reconstructed by D/A 1, A/D 1 by D/A 2, A/D 2 by

D/A 3, and A/D 3 by D/A 0.  In this program, latent reads are handled

by altering this sequence of the address set-up, read A/D, and write D/A

to take advantage of the latency.

Example 3

~~~~~~~~~~

In the program shown in Program Listin 3, the average of the four A/D

converters is computed, and the result is output to the four D/A converters.

After the average is computed, it is written to each of the four D/A

converters seperately. This part can be significantly speeded up by

enabling all four D/A converters (with the address 0x0F) and then

performing a single write.

Example 4

~~~~~~~~~~

Program Listing 4 illustrates the use of the Control/Status port. This

port is read during the execution of the interrupt service routine.

The switch settings are read, and one of four different, independent

programs is executed.  The LEDs are written by the DSP16 device to

reflect the switch settings.  In order to reduce the size of the listing,

only the start-up and control part of the program is shown.

Summary

~~~~~~~~

Table 3 summarizes the program size and execution time of the interrupt service

routines for the programs shown in Program Listings 1, 2 and 3.

Table 3.  Summary of the Interrupt Service Routines

+-----------------+-----------+-------------+----------+

| Program Segment | ROM Usage | Instruction |   Time   |

|                 |  (words)  |   Cycles    | (60 MHz) |

+-----------------+-----------+-------------+----------+

|   Example 1     |    23     |     36      |  1.18 us |

|   Example 2     |    17     |     26      |  0.94 us |

|   Example 3     |    36     |     52      |  1.88 us |

+-----------------+-----------+-------------+----------+

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=