ThinLift v1.0

I.    FILES IN THE THINLIFT.ZIP FILE

Install THINLIFT in your HP.

XXX - formerly XPURGE which makes fast deleting of directories.

XPURGE can be found on Goodies Disk 7 for HP48 in UT dir.

THINLIFT.WQ1 - Quattro PRO spreadsheet.

THINLIFT.WK1 - Lotus spreadsheet.

OUTLIERS.TXT - Text file explaining outliers.

THINLIFT.DOC - This file.

THINLIFT.INF - Information for distribution to internet.

THINLIFT     - Binary file for HP48.

/* The spreadsheet program was written by Reynolds Toney and Danny Gierhart. */

Will this work on an hp48gx?  Yes.... hp48sx? Yep..... THINLIFT  is the

binary directory to install on your HP48SX/GX.  OCF.ASC is the ascii

encoded file that can be converted to binary or print out on the pc.

THINLIFT.DOC is what you are reading now.  THINLIFT.INF is just an

information file for Chris Spell at ftp site of wuarchive.wustl.edu (hp

files in systems dir.).  Look there for some of my other files and many

other contributors.

II.    SETTINGS

1.   LS is left shift key.

2.   ALPHA is the alpha key (just below ENTER).

3.   3 LS CST FIX or 3 ALPHA ALPHA FIX ALPHA ENTER

/* Fixes decimals to three places though the programs will do this

anyway. /*

4.   As above but set SYM off /* no white box with SYM */

5.   As above (press NXT NXT) with DEG on and XYZ on /* with box */

6.   ON++ on ON+- to darken/lighten display.

7.   VAR for Var Menu.

III.   EXAMPLE

A.   VAR           /* Make sure var menu is displayed            */

B.   THINLIFT      /* Goes to THINLIFT directory                 */

C.   CST           /* Enters Custom Menu                         */

D.   TRIAL         /* Calibrate Nuclear Gauge with Roadway Cores */

E.   :Dir Name?:   /* Press ALPHA twice and enter a directory to store

this in say CANA14, Press ALPHA and then down

arrow*/

F.   :Max.T. at JMF: 2.471

G.   ENTER

H.   Press Down Arrow to edit 10x4 matrix of Nuclear Gauge Data

/* GO-> with box should be on */

2000 138.1 137.9 134.4 ENTER   /* Pres SPC key between entries */

140.3 139.8 137.7 139.4 ENTER  /* ENTER is press enter key     */

143.9 143.2 141.7 145.4 ENTER

138.3 139.2 139.5 139.6 ENTER

142.6 143.1 142.9 142.5 ENTER

137.3 138.0 135.3 137.3 ENTER

144.4 140.0 138.8 145.4 ENTER

143.3 141.1 142.7 143.0 ENTER

139.2 140.3 137.9 137.6 ENTER

134.4 134.3 135.2 134.8 ENTER

/*You can press ENTER here to put matrix on stack or.....  LS ON (which is CONT)

IF you press ENTER you can edit what you just entered by pressing the down arrow

key else go ahead and now press CONT (LS key and then ON key)    */

/* A short cut could have been used for this example, see below: */

/* For testings sake you can use the matrix A that is the same as the one above

that you normally create for calibration.  Delete the zero matix (Press backspace

key) and then ALPHA A ENTER.  Now CONT to continue.  */

I.   You will see a note displayed.

You enter data for Roadway Cores.  Air Wt., Water Wt., and SSD

saturated surface dry Wt.  Down arrow and enter data as below:

3256.5 1847.3 3281.1 ENTER

3131.7 1799.8 3174.2 ENTER

2512.9 1453.6 2525.4 ENTER

3007.2 1704.7 3020.4 ENTER

3490.4 2009.5 3503.4 ENTER

2530.8 1456.8 2576.0 ENTER

2430.0 1388.1 2433.5 ENTER

3239.6 1854.9 3259.4 ENTER

3005.1 1704.3 3018.9 ENTER

3339.1 1883.4 3361.4 ENTER CONT

/* or.... for shortcut B is the matrix above.  Backspace to delete zero matrix and

ALPHA B ENTER CONT to pull up the same matrix as above. */

J.   One Minute Please... displays.  It takes about one minute or a

little more to do the computations.  Results below:

5: [[ 1.000 .........]]

4: Avg. NucBSG: 2.238

3: BSG. Calc:   2.295    /* Avg. Corrected Nuclear BSG */

2: Max. Theo.:  2.471

1: Rdwy. Den.: 92.892

/* Up arrow and VIEW to view the densities.  Col1 (Column one) is LOT number and

COL2 is density of each Roadway Core and COL3 is density of each Calculated

Nuclear Gauge (BSG. Calc.),  The Corebsg contains the average core bulk S.G. */

/* ON ON RS backspace key to CLR (clears display).  EDIT NEWL (NEWLOT) and INMA

(INmax) are then on menu. */

K.   EDIT - should you wish to edit trial calibration matrix data.

/* IT is like Trial only you do not create another subdirectory and

you edit the matrix that you entered in TRIAL Steps I and j. */

NEWL - Put 'NEWL and you'll see the NEWL is 'NEWLOT' ON key to

continue if you checked NEWL.

INmax - Value of Maximum Theoretical Specific Gravity at JMF

L.   NEWLOT

:Dir Name?: LOT1  /* ALPHA ALPHA LOT1 ALPHA down arrow key */

:Max.T. at JMF?: 2.471 /* or...  INmax RS EVAL and ENTER   */

/* RS EVAL is ->NUM                 */

M.   Text information is displayed.  Down arrow to edit zero matrix.

140.3 138.1 137.9 134.4 ENTER

140.3 139.8 137.7 139.4 ENTER

143.9 143.2 141.7 145.4 ENTER ENTER CONT

/* Like Steps H and I, backspace key ALPHA C ENTER CONT if you want the matrix

above to test */

N.   One Minute Please... displays.  It takes about one minute or a

little more to do the computations.  Results below:

5: [[ 1.000 .........]]

4: Avg. NucBSG: 2.246

3: BSG. Calc:   2.300    /* Avg. Corrected Nuclear BSG */

2: Max. Theo.:  2.471

1: Rdwy. Den.: 93.063    /* Like Step J only line 5 lists just the

LOT number and density for that LOT */

O.   EDITL ('EDITLOT') and INMA ('Inmax') are now displayed

EDITLOT lets you edit your nuclear gauge matrix data from Step M.

INmax is just there for easy access to the Max. Theo. S.G. (See Step

L) last part.  This is the custom menu so if you come to this

subdirectory of {THINLIFT CANA14 LOT1} and press CST key you will

be in the custom directory.

III.   GENERAL RUNNING

A.   Enter into the {THINLIFT} directory

B.   To start a new TRIAL Calibration Press CST key.  TRIAL will

be displayed.  Continue as in Part III.

C.   To start a new LOT say LOT2 for TRIAL in directory {THINLIFT CANA14}

enter into the {THINLIFT CANA14} directory.  Press CST key.

EDIT NEWL (NEWLOT) and INMA (INmax) are displayed.  Continue as in

PART II Step L - O.

D.   To edit a LOT say LOT1 enter {THINLIFT CANA14 LOT1} directory.  This

is usually done from the VAR menu.  Once you make it to that

directory press CST key.  EDITL (EDITLOT) and INMA (Inmax) are

displayed.  Press EDITLOT and continue as in PART II Step O.

E.   HOUSE CLEANING - If you want to delete a TRIAL Calibration and all

LOT subdirectories below, then in the THINLIFT directory place

'CANA14' with quotes or {CANA14} with brackets on stack and XXX or

PGDIR on STACK.  XXX is XPURGE a fast directory delete utitlity.

IV.    PROGRAM LISTING

Calc     - Main sub program for MEAN and StdDev calculations.

size     - Variable value for number of rows.

INnuc    - 10X4 matrix for input of nuclear gauge readings No. 1 to 4.

Inrc     - 10X3 matrix for input of roadway core data.  COL1 for air wt.,

Col2 for water wt. and Col3 for Saturated Surface Dry wt.

Inmax    - Input value for Max. Theo. S.G. at JMF

SigmaDAT - Sigma is the greek symbol.  Statistics matrix.

STOREIT  - Stores computed values to OUTmat.

OUTmat   - 10X7 matrix. Each column:  Mean/average for row of INnuc,

Standard Deviation for row of INnuc, T1 to T4, unit wt. (pcf)

adjusted.

Tcalc    - Sub program for Calc.  Puts T1 to T4 in colums 3-6 of OUTmat.

LIER     - Computes outliers, sub of Calc.

NUMREAD  - No. reading of nuclear gauge.

NO1 to No4 - Acutal No. reading of nuclear gauge.

T1 to T4 - T value if last/current row.

sdmat    - Std. Dev. matrix 1X10.

meanmat  - Mean matrix 1X10.

BULKSG   - sub of Calc.  Finds OUTbsg 10X3 matrix.

OUTbsg   - 10X3 matrix.  Nuclear S.G., Core Bulk S.G., Bulk S.G. (Calc.).

LRcalc   - sub of Calc.  Puts calculated/corrected nuclear gauge S.G.

into row 3 of OUTbsg.

TRIAL    - First custom menu program to run.

SUMMARY  - Summary of results for calibration sub.

SUMLOT   - Summary for LOT.  Sub of EDITLOT.

EDITLOT  - Replaces EDIT for LOT calculations.

NEWLOT   - Replaces TRIAL for LOT calcutlations.

DENtrial - Sub of SUMMARY densities (Calc.) for TRIAL.

DENlot   - Sub of SUMMARY densities (Calc.) for LOT.

LDEN     - 3X2 zero matrix used by DENlot.

NEWLOT   - LOT equivalent for TRIAL (New LOT).

AVGLOT   - value for average lot.

Avgnucbsg - value for average nuclear B.S.G.

Bulksgcalc - value for calculated B.S.G.

Rdyden   - average roadway density.

A        - Test matrix for nuclear gauge readings.  Used to test TRIAL

B        - Test matrix for roadway core data.  Used to test TRIAL.

C        _ Test matrix for nuclear gauge readings.  Used to test LOT.

Z3X4     - 3X4 zero matrix.

Z10X3

Z10X4

CST      - Custom Menu variable.

WORKING  - Text message.

EDIT     - Edits TRIAL data.

ENT2n    - sub of TRIAL for new data entry.

ENT1o    - sub of EDIT for old data entry from TRIAL.

TXT1 to TXT3 - Text messages.

TAVG     - Last T row average.

sdmat    - Std. Dev. matrix

meanmat  - Mean matrix

NOTES:  I hope you enjoy these programs.

Several programs that I use and have posted are similar to commercial

cogo type routines.  These are free for your collection.  Try AREA,

KAREA, COINTERCEPT (in dos cointerc), HCURVE, FRACDC, VC, VCRV,

TRIANGLS (/\S or #_S on the pc)or triangle.zip, and VCM.  OCF.ZIP (open

channel flow).

Most can be found at the ftp site of wuarchive.wustl.edu in the

systems/hp48 and below directories.  Some are backed up from a while

back.

VCM is the latest version of a solver for multiply vertical curves.

For a short single vertical curve I recommend my VC program.  Second,

I would recommend VCRV (Dewayne Cushman wrote this one as an

enhancement for VC that he modified from VC (it gives extra data).

Lastly, and the longest is the VCM.  It works just like the VC and VCRV but

it can do any number of curves, even single.

BIO: - Kenneth Hobson wrote this document for you.  This program was done by

Kenneth Hobson on 03/08/95.  Ken graduated in Spr.'93 from OU and is an Intern I

Civil Engineer for the Oklahoma Department of Transportation.

Regards,  Kenneth Hobson          kenneth.hobson@torii.nova.com

kenneth.hobson@s-box.misc.uoknor.edu

work  internet address: kenneth.hobson@dsibm.okladot.state.ok.us

or: umtr121@dsibm.okladot.state.ok.us

Work phone 405-521-2677