Horizontal Curve v1.0

I.    FILES IN THE HCURVE.ZIP FILE

HCURVE    (binary directory for hp) contains:

HCDI

HCDL

HCRVE

TOFFSET

DEFL

HCURVE.DOC (this file your reading)

HCURVE.ASC (HCURVE in ascii format for easy reading)

HCURVE.INF (information file)

II.    EXAMPLE

1.  A 6 degree curve (english units)

Intersection angle of 3.5 degrees.

A.  In HCURVE directory press HCDI

Notice DI means you know degree of curvature and Intersection

angle.

a.      :Degree: 6 (press down arrow once and right arrow twice)

b.      :Int. Ang.: 3.5 (Note: you can enter as HMS with HMS->)

c       ENTER  (press enter key)

d.      8: Degree: 6.00   (your output, press up arrow to view)

7: Int. Ang.: 3.5  (Int. Ang. Part not shown if you used

3.3000 HMS-> method)

6: Len. Curve: 58.33 (ft. of course)

5: Radius: 954.93

4: Tan. Offset: 29.18

3: Lg. Chord: 58.32  (Long Chord)

2: Ext. Dist.: 0.45  (E)

1: PI to Mid.: 0.45  (M, press ON to end view)

B.  HCDL

a.      :Degree: 6

b.      :Length: 58.33  (Note: that was the Len. of Curve in A.

c.      ENTER

d.      8: Degree: 6.00    (16 is what you had in A. if you did)

7: Length: 58.33   (not clear the display prior to B.)

6: Int. Ang. Deg.: 3.4998 (not 3.5 degrees because of

5: Radius: 954.93         (length was rounded to 58.33)

4: Tan. Offset: 29.17

3: Lg. Chord: 58.32

2: Ext. Dist. 0.45

1: PI to Mid: 0.45

C.  HCRVE   (equations)

a.      6 D    (store 6 degrees in variable D)

b.      3.5 I  (store 3..5 degrees for intersection angle)

c.      LS L   (left shift L to solve for Length of curve)

L: 58.33 (right shift variable to view)

d.      NXEQ

e.      LS R    (solve for Radius)

R: 0.29 (incorrect but continue)

f.      NXEQ

g.      LS R    (Now radius of ok)

R: 954.93

h.      NXEQ

i.      LS T    (solve for Tangent offset)

29.18

j.      NXEQ

k.      LS LC   (solve for long chord)

LC: 58.32

NXEQ

l.      LS E     (solve for E, external distance)

E: 0.45

NXEQ

m.      LS M      (solve for M, PI to Mid.)

M: 0.45

NXEQ

(loops back)

n.      LS ON     (or CONT to continue, cleans up and exits)

o.      CLR       (clear display if you like)

D.  TOFFSET and DEFL calculate small tangent offsets and deflections

I'll not go into that.  It is like C. in a fasion.

NOTES:  I hope you enjoy these programs.

Several programs that I use and have posted are similar to commercial

cogo type routines.  These are free for your collection.  Try AREA,

KAREA, COINTERCEPT (in dos cointerc), HCURVE, FRACDC, VC, VCRV,

TRIANGLS (/\S or #_S on the pc)or triangle.zip, and VCM.

OCF.zip (open channel flow) and thinlift.zip (nuclear gauge density) are

others that I have written.

Most can be found at the ftp site of wuarchive.wustl.edu in the

systems/hp48 and below directories.  Some are backed up from a while

back.

VCM is the latest version of a solver for multiply vertical curves.

For a short single vertical curve I recommend my VC program.  Second,

I would recommend VCRV (Dewayne Cushman wrote this one as an

enhancement for VC that he modified from VC (it gives extra data).

Lastly, and the longest is the VCM.  It works just like the VC and VCRV but

it can do any number of curves,

even single.

BIO: - Kenneth Hobson wrote this document for you.

Ken graduated in Spr.'93 and is and Intern I Civil Engineer for the

Oklahoma Department of Transportation.  405-521-2677

These programs are useful for survey and bridge work.  They would have

been nice to have had when I took SURVEY, Hydraulics, .....

Modify anything you like.  Please send me a copy if you do modify these and

document it and I will upgrade this version of HCURVE.

khobson@telepath.com

Regards,  Kenneth Hobson          kenneth.hobson@torii.nova.com

kenneth.hobson@s-box.misc.uoknor.edu

work  internet address: kenneth.hobson@dsibm.okladot.state.ok.us